13. Lost communications procedures

The MAV relies on a heartbeat signal from the ground control station to indicate a valid control link. The heartbeat signal must be periodically received/processed by the MAV. The default time for return home or rally point is 30 seconds. The operator can decrease the time down to 15 seconds if engagement of the return home (lost link) mode is desired to occur quicker. Loss of the communication link should result in the MAV executing its lost link procedures. The operator also has the option to power up a second ground control station and attempt to establish a communication link with the MAV. It would require multiple failures for the MAV to lose its communication link and to fail to engage lost link, or fail to respond to a second ground control station.