ONE SYSTEM GROUND CONTROL STATION

The One System Ground Control Station (OSGCS) serves as the Aerial Vehicle (AV) operator interface. It is the central point from which the Air Vehicle Operator (AVO) and Mission Payload Operator (MPO) coordinate all mission and communication functions. The OSGCS provides near-real time control, near-real time mission progress, video display, and system function monitoring. The OSGCS provides operator interface through on screen displays (including point and click buttons, window and sliders), foot switches, knobs and joysticks. AV control modes optimize payload control while the AV maintains a loiter point of interest (stare point). Video is displayed in near-real time, or recorded for detailed analysis later. All information can be transferred to approve outside sources through the Tactical Local Area Network (TACLAN) interface.

The OSGCS contains the (Multiple Unified Simulation Environment) MUSE rack which is designed to simulate AV operational missions for training the AVO and MPO personnel. It is for training purposes only. AV flight data is simulated to familiarize both operators with the capabilities of each station. The MUSE rack computers also record uplink and downlink telemetry during mission execution.

The OSGCS consist of a High Mobility Multi-Purpose Wheeled Vehicle (HMMWV) transport vehicle with a mounted modified S-788 shelter assembly, enclosed hardware, software, crew support equipment, and external support equipment. The OSGCS includes power generating and distribution equipment necessary to support the performance of its mission. The OSGCS is capable of supporting its crew in planning the missions and interpreting the data for the supported types of AVs.

Figure 1.   ONE SYSTEM GROUND CONTROL STATION Vehicle.

Figure 2. Flat Panel Display (AVO, MPO).

Figure 3. Center Flat Panel Display

The CFPD (1A8) is located in the center of the upper workstation. Images displayed on the CFPD are switch able between the AVO and MPO. Switching is accomplished by the CFPD Video Switch (1A13S4). The CFPD is a Active Matrix Liquid Crystal Display (AMLCD) with a wide viewing angle that incorporates amorphous silicon Thin Film Transistor (TFT) technology. A 1.1 mm glass plate is bonded to the front of the panel to give improved optical performance. Front bonding also adds the necessary strength to allow the AMLCD panel to survive substantial shock and vibration in rugged environments and provides the heating mechanism for the CFPD for cold startup. The FPDs are capable of displaying full color and comes with digital control buttons on the front panel.

Figure 4.  9 Inch Video Monitor