Emergency Procedures

Written emergency procedure checklists for the UAS operators have been established for lost link, lost communications, mission abort, engine failure, and electrical failure.  Additional emergency procedures are covered in the Army MQ-5B Technical Manual.

The pilot has the capability to override the mission plan and “manually” control the vehicle heading and altitude or if necessary to terminate engine power and land the aircraft using a parachute.  The pilot may also exit override control and return the aircraft to autonomous operation by directing the vehicle to a mission-planned waypoint.  The pilot can command subsystem functions such as lights, strobe, ATC radio, radar altimeter, IFF, etc., if needed while the UA is airborne.

The pilot will ensure that appropriate ATC facilities and Zoom Control (Exercise Controller) are appraised of the emergency.  The UA operator will fly back to WAAF using the COA corridor(s).  If the emergency is flight critical requiring immediate recovery, the pilot will change the transponder code to 7700 if possible.

The Hunter B COA corridor was designed over primarily uninhabited or sparsely populated areas to mitigate a potential issue with ground personnel or structures in the highly unlikely event the UA needed to land immediately.

In-addition to the above; procedures established in the Letter of Agreement between the Directorate of Plans, Training Mobilization and Security, Aviation Division Fort Stewart, Georgia and Commanders of the 224th Military Intelligence Battalion shall be followed.