##### General Safety Rules and Flight profiles

## Brennan Field

Location

- MGRS approximate location 11SLT 2954 9287

- Field elevation is approximately 600 MSL.

# Rules

- Field is only approved for Raven, Wasp, and Batmav SUAV flight-testing only.

- 30db attenuation is required for Raven.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 3 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 600 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1000 feet MSL or 400 feet AGL.

- Flights should not go below 700 feet MSL or 100 feet AGL unless taking off or landing.

-     NOTAM’s need to be filed with FSS prior to any flights.

- The Flight Coordinator or Lead SUAV operator is responsible for filing   NOTAM’s.

- Navigation waypoints must be entered into the GCS prior to flight.

Range     Bearing

Waypoint A 11SLT 2894 9287     600m       269

Waypoint B 11SLT 2889 9287     650m       269

Waypoint C 11SLT 2924 9235     600m       209

Waypoint D 11SLT 2921 9231     650m       209

Waypoint E 11SLT 2904 9283     500m        266

Waypoint L will be set to 11SLT 2944 9285

-     Flights will be terminated for adverse wind and weather conditions.

#### Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL.

- Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flight performance tests must remain in the airspace defined by these MGRS coordinates. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SLT 2954 9287).

Range      Bearing

1) 11SLT 2967 9290        132m          74

2) 11SLT 2993 9255        500m        128

3) 11SLT 2926 9257        420m        222

4) 11SLT 2927 9221        700m        200

5) 11SLT 2894 9251        700m        239

6) 11SLT 2884 9289        700m        271

7) 11SLT 2938 9295        174m        295

-  The SUAV will always be landed into the wind.

-  (With a west and calm wind condition) a downwind, base, and final will be

performed using left turns. With an east wind condition a straight in approach

will be executed. All approaches to land will be made from the southwest.

### Upper Field Waste Management

## Location

- MGRS approximate location 11SLT 3416 9813

- Field altitude is approximately 1000 feet MSL.

## Rules

- Field is approved for Raven, Swift, Pointer, Wasp, Batmav, Puma, and Dragon Eye.

- 30db attenuation is required for Raven, Pointer, Puma, Swift, and Dragon Eye.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 3 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 1000 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1400 feet MSL or 400 feet AGL.

- Flights should not go below 1100 feet MSL or 100 feet AGL unless taking off or landing.

-     NOTAM’s need to be filed with FSS prior to any flights.

- The Flight Coordinator or Lead SUAV operator is responsible for filing   NOTAM’s.

- Navigation waypoints must be entered into the GCS prior to flight.

###### South-East Navigation pattern

Range     Bearing

Waypoint A 11SLT 3415 9758      550m      179

Waypoint B 11SLT 3416 9748      650m      179

Waypoint C 11SLT 3463 9786      550m      119

Waypoint D 11SLT 3471 9780      650m      119

Waypoint E 11SLT 3457 9842      500m        54

(Waypoint L = Waypoint H)

###### West Navigation pattern

Range     Bearing

Waypoint A 11SLT 3360 9813     550m       269

Waypoint B 11SLT 3350 9813     650m       269

Waypoint C 11SLT 3388 9765     550m       209

Waypoint D 11SLT 3383 9757     650m       209

Waypoint E 11SL T 3457 9842    500m         54

Waypoint L = Waypoint H

- Flights will be terminated for adverse wind and weather conditions.

#### Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL. Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flights must remain inside a 0.7km radius of the HOME coordinate. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SLT 3416 9813).

-    The SUAV will always be landed into the wind.

# Lower Waste Management

#### Location

- MGRS approximate location 11SLT 3542 9728

- Field altitude is approximately 1000 feet MSL.

#### Rules

- Field is approved for Raven, Pointer Swift, Puma, Wasp, and Batmav.

- 30db attenuation is required for Raven, Pointer, Puma, and Swift.  20db attenuation is required for Wasp and Batmav.

- Un-attenuated flights are possible if SUAV frequencies in use are coordinated thru the Flight Coordinator.

- No more than 2 SUAV’s should operate simultaneously.

- No flights should exceed a distance of 0.7km from the SUAV operator/observer.

- An accurate boot up altitude of 1000 feet MSL + or – 50 feet must be verified prior to launch.

- Flights not to exceed 1400 feet MSL or 400 feet AGL.

- Flights should not go below 1100 feet MSL or 100 feet AGL unless taking off or landing.

- Navigation waypoints must be entered into the GCS prior to flight.

Range     Bearing

Waypoint A 11SLT 3494 9756     550m       299

Waypoint B 11SLT 3485 9761     650m       299

Waypoint C 11SLT 3494 9700     550m       239

Waypoint D 11SLT 3485 9696     650m       239

Waypoint E 11SLT 3491 9728     500m       269

Waypoint L = Waypoint H

- Flights will be terminated for adverse wind and weather conditions.

Flight Profile

- The SUAV will always be launched into the wind.

- After launch the SUAV will be flown to an altitude of 300 feet AGL. Once the SUAV has reached its climb altitude the SUAV operator can complete the flight performance test at their discretion.

- All flight performance tests must remain in the airspace defined by these MGRS coordinates. (Ranges and bearings are in reference to the GCS HOME coordinate of 11SLT 3542 9728).

Range      Bearing

1) 11SLT 3570 9757         400m          44

2) 11SLT 3570 9713         300m        114

3) 11SLT 3484 9689         700m        235

4) 11SLT 3474 9703         700m        249

5) 11SLT 3472 9727         700m        269

6) 11SLT 3480 9764         700m        299

7) 11SLT 3492 9777         700m        314

-    The SUAV will always be landed into the wind.