Ground Obervers

Ground observers are trained members of the flight crew. Ground observers are trained in airspace awareness and the requirement to keep the UA deconflicted from all other airborne operations.

The Class D airspace over WAAF is sterilized by Wright Tower prior to any departure or arrival. The Mission Commander (MC) and ground observers however visually assess the surrounding airspace and are capable of notifying the UA operator in the event of any potential conflict.  In addition to the MC and ground observers, the UA is provided air traffic services and advisories by Wright Tower ATC. The operator will take appropriate action to avoid other airborne traffic as instructed by ATC or upon notification by an observer on headset.  Should another aircraft's flight path appear to take it into the flight path of the UA, the UA will alter its flight path away from the other aircraft.

The lateral and vertical limits for all ground observers are 1 NM, Surface to 2000’ AGL. The primary ground observer is the Non Flying External Operator (EO) who operates from the runway with direct communications with ATC. The EO is assisted by 3 crewmember ground observers on the flight line and one place midway between R-3005 boundary and WAAF runway. The ceiling of the Class D airspace is 2500’ MSL.

Ground observers use headset communications on a common net with the Mission Commander and the Ground Control Station.

Observers will only monitor one (1) UA at a time and will not operate the aircraft while performing observer duties.

Ground observers include trained military personnel and NG contractors under contract with Department of the Army.

Patrol/chase aircraft

Chase aircraft pilots must not concurrently perform either observer or UA pilot duties along with chase pilot duties. Observers onboard a chase aircraft must keep visual contact with the UA at all times.  To the extent consistent with the safety of the chase aircraft, the chase aircraft should be operated within one mile laterally and 3000 feet vertically from the UA.

Any visual observer, radar monitor, or sensor operator charged with providing collision avoidance for the UA must have direct communication with the UA pilot.  The frequency 260.9 will be the primary frequency for this purpose.

The Coast Guard has an FAA exemption (5231B) to conduct aerial intercept 	procedures.  Zoom Control will direct the intercept with the Hunter on 260.9 prior 	to exiting R-3005 and W-157 on the return flight.  The Coast Guard will fall into 	the shadow position of approximately 300-500' lateral offset from the Hunter’s 	left rear quarter, 30-45 degrees aft of the UA and with a 200-300' step up.

Chase aircraft observer will have direct communications to the UA pilot.  All personnel on board the chase aircraft will provide see-avoid capability, however, the on-board observer will be responsible for relaying any traffic and actions required of the UA pilot to avoid known traffic.

The on-board observer will meet all the FAA and DoD requirements to perform the functions of an observer to include medical and Part 91.  The observer will also be a qualified MQ-5B operator.

Only one (1) UA will operate in the NAS under this COA.

The chase aircraft and the UA will be treated as a flight with MARSA.

Cameras

The Hunter MQ-5B utilizes a SPA 770 with two EO/IR sensors. The Stabilized Platform Assembly (SPA) 770 primary use is to provide either IR or Television (TV) real time imagery down linked from the AV to GCS.  The SPA 770 cameras have focus and zoom capability.

Sensor #1 – Electrical Optical:

Type: HxV, 320 x 240 IR

Resolution: 480 TV lines

Fields of View:  Continuous Zoom Capability (20-280mm w/ 14:1 ratio)

Sensor #2 - Infrared:

Camera Type:  HxV, 320 x 240 IR

Resolution: 480 TV lines

Fields of View: 3 Steps

Wide 6.5 degrees Horizontal x 12.5 degrees Vertical

Medium 6 degrees Horizontal x 2.7 degrees Vertical

Narrow 0.92 degrees Horizontal x 0.68 degrees Vertical

Video can be displayed on a dedicated LCD screen at the operator station or as an inset on the control monitor.   Additionally, video can be piped out of the OSGCS to an outside monitor.

Cameras & displays have a high reliability rate.  Communications link is redundant. (Primary and back-up Links)