What happens when command link is lost?

The operator can pre-program one of three options.  Option 1 is Land Now: The air vehicle executes the Autoland function immediately.  Option 2 is Go to Rally: The air vehicle flies to the specified rally waypoint and executes the Autoland function at the rally waypoint.  Option 3 is Finish Flight: The air vehicle will continue to fly the current waypoint loop until cycle time expires, then go to the rally waypoint and autoland. Default is Land immediately.  Most operations call for the rally point to be the same as HOME.  Go to Rally is the default LOL setting which in fact forces the vehicle to HOME.

How does vehicle respond if link is never re-established?

In all modes, the end result is that the air vehicle has landed.

How does the vehicle recognize that loss of command link has occurred?

The Air Vehicle monitors the number of link packets that it receives from the GCU.  The frame rate is roughly 10 per second.  If the Air Vehicle receives zero packets per second, it would recognize that a loss of link has occurred. There are two numbers displayed on the GCU that describe the quality of the link.  The first number is the number of packets sent up by the GCU (number the aircraft received), the second number is the packets sent down from the Air Vehicle (number the GCS received).  A  9-10 status on both is good.  Anything down to about 2-3 on either up or down still results in a working up and down link.  The numbers are directly related to the link bar indication.  The bar is basically a 0 to 10 scale.  The link bar indicates the least of the two RT values.  So a 10/5 or a 5/5 would show up as a 50% link bar on the aircraft display.  A 10/5 also indicates a strong uplink, and downlink with some loss.  A 5/5 indicates some uplink loss, but still a good downlink.

How does the UAV operator in the ground control station recognize loss of command link has occurred?

The Link Status Bar on both the Aircraft and Mission Controller is a continual display of the quality of the uplink and downlink.  If the command link is lost, a warning message (clock icon) is displayed on the GCU controllers.  Air Vehicle will send a loss of link “En Route Rally” message to the operator.