Communications

VOICE COMMUNICATIONs.  The One System Ground Control Station (OSGCS) is equipped with UHF/VHF/FM radios for local airfield and enroute air traffic control communications.  The system is equipped with an ARC-210 voice radio repeater, which provides for VHF (118-136 MHz) and UHF (225-400 MHz) real time voice capability between the pilot and ATC agencies.  The pilot is capable of changing frequency, as appropriate, while the UA is in-flight.  Telephones are available for backup communications with ATC in the event of lost data link with the UA or radio relay failure.

The OSGCS also includes an AN/VRC-103 UHF/VHF radio set and is also capable of communicating with tactical and fixed air traffic control agencies. It is made up of an RT-1796/PRC Radio (AN/PRC-117F (V)) and the AM-7588/VRC-103 Vehicle Adapter Unit (VAU) which houses a power amplifier. This radio supports interoperability of the Hunter system with military and commercial air traffic controllers (ATC) by providing a voice link from the OSGCS to these agencies.

All flights will be in communication with ATC.  Voice communications between the pilot or chase aircraft and ATC shall be as directed by the affected ATC facility.  Direct voice communication between the pilot or chase aircraft and ATC will be accomplished through UHF/VHF LOS or the relay system on-board the aircraft, with a backup land line between the GCS and ATC.  Frequency changes from ATC will be performed in the same manner as manned aircraft operations.  Direct voice communication will be the same as manned aircraft operations.  Direct voice communication with all affected ATC facilities will be accomplished, unless an affected facility authorizes another means of communication between the pilot or chase aircraft and that facility’s control positions.

DATA LINKS.  The MQ-5B system provides continuous command and control communications from the GCS to the UA via LOS.  These links allow dynamic vehicle control and command updates by the pilot and also provides continuous monitoring of the health and status of the vehicle.

MANEUVERABILITY.  MQ-5B turns are performed at route turn points or when initiated by the pilot.

RESPONSIVENESS.  MQ-5B will respond to override heading and/or altitude commands from the pilot.

AUTONOMOUS CONTROL.  MQ-5B can be set up to fly autonomously by the aircrafts on-board computers.  The navigation mission plan consists of a series of waypoints defining the latitude, longitude and altitude through which the aircraft will be guided.