  - Lost Link Procedures

    - General

      - If the aircraft loses link, the PIC will immediately inform the controlling agency and the appropriate observer of the aircraft’s expected flight path then attempt to regain link.

      - If the aircraft regains link the PIC will cancel the lost link status and coordinate with the controlling agency and the observer.  If a deviation has occurred the PIC will return the aircraft to the previous clearance.

      - The observer will maintain visual and transmit position updates to the PIC.  The PIC will be responsible for coordination with controlling agencies.

      - If link is not regained the aircraft will proceed as specified below.

    - VCV Class D

      - The ground observer will reposition as necessary to maintain sight of the aircraft and notify the PIC of any hazards.  The PIC will relay the hazards to ATCT on an information only basis.

      - The aircraft will climb to 5000 MSL and proceed to the lost link loiter to the northeast of the airfield until link can be regained.

      - If link cannot be regained a chase plane will be launched.  The chase plane will join up with the aircraft and monitor the UAS as it exits Class D airspace up to its termination orbit in the R-2508 complex.

    - Transit to R-2508 complex

      - If link cannot be regained the aircraft will enter R-2508 and proceed to its termination orbit

      - If link can be regained the PIC will cancel the lost link status and proceed on course.

    - Transit to R-2501 complex

      - If link cannot be regained the aircraft will proceed to R-2501 with the chase aircraft.  The aircraft will loiter in R-2501 until link is regained.  If link is not regained, this will be its termination orbit.

      - If link can be regained the PIC will cancel the lost link status and continue operation in R-2501.

    - Established in R-2508

      - The aircraft will loiter in assigned airspace while the PIC attempts to regain link.

      - If link cannot be regained the aircraft will transit to its termination orbit.

    - Established in R-2501

      - The aircraft will loiter in R-2501 while the PIC attempts to regain link and the chase aircraft launches.

      - If link cannot be regained the aircraft will remain in R-2501 at its termination orbit.

    - Transit from R-2508 or R-2501 complex

      - The aircraft will proceed with chase to VCV Class D and loiter in the Class D airspace.

      - If link cannot be regained the aircraft will exit VCV with chase and transit to R-2508 in accordance with (3) above.