Launch/recovery

The Hunter MQ-5B takeoff and landings are manually conducted under the visual and data link control of a trained Vehicle Operator (VO) and External Operator (EO) team from the One-System Ground Control Station.  The VO will establish radio contact with ground control and give departure information, request taxi clearance and authorization to occupy the active runway.  Once cleared for taxi, the primary EO will taxi aircraft into position with the assistance of the UAS ground crew while the Backup EO sets up arresting gear across the runway.  Once in position and gear is setup the EO will contact tower for final departure clearance.  Once airborne the non-flying EO will move back to the holding area to receive the Air Vehicle (AV) control.  After control transfer between EO’s is complete the non-flying EO clears all equipment and moves to the holding area.  The EO will climb to 2000’ AGL in compliance with ATC clearances and hand off flight control to the VO, who will continue flight into restricted airspace as per instructions from ATC.  Recovery operations are conducted in reverse.  The primary launch and recovery are pre-coordinated with the control tower at WAAF via direct, FM, VHF, or UHF radio communications. Aircraft are cleared for takeoff and landing by Wright Tower.  Only one runway (6R/24L) will be used for Hunter UAS operations at one time.