Lost Link Procedures:

Note: There are four possible lost-link settings; Land Now, Finish Flight (NAV mode), Finish Flight (all other modes), and Go to Rally. The two most commonly used procedures are as follows. Land Now -- the Raven can be programmed to auto land immediately (within 3 seconds) when the loss-of-link condition is detected on the GCS. The second, Go to Rally setting is for the Raven to auto land at a predetermined rally point (normally over the GCS). In the unlikely event of errant or dangerous behavior of the Raven, the operator has the option to shut down the motor forcing an auto land. Specific procedures follow:

A. If loss of link occurs at anytime during the mission, the Raven will fly to a predetermined rally point at a predetermined altitude (typically 300 ft AGL). The rally point is set as the same point as the location of the GCU. During the return flight, if link is reestablished, the pilot will make a determination on whether to continue with the mission or land the aircraft.

B. If link is not reestablished, the aircraft will return to the rally point and autonomously set itself into “Autoland”.