MQ-1 Communication

The MQ-1 will be flown by both a launch and recovery element (LRE) at SCLA or a mission control element (MCE) at March Air Reserve Base.  The LRE’s ground control station has two UHF/VHF ARC-210 radios that transmit/receive within line of sight from SCLA.  The MCE crew’s ground control station at March ARB can transmit/receive on the aircraft’s airborne ARC-210 that is also UHF/VHF capable.  In case of lost link, the MCE crew would not be able to transmit/receive from the aircraft’s radio, but they have telephone access to call a controlling agency or the LRE who can transmit from their ground station.  The LRE is always able to transmit/receive since their radios are ground-based.