| Launch/Recovery Procedures: |  |
| --- | --- |
| Note: In accordance with the Department of the Navy, Memorandum of Understanding between Commanding Officer, Naval Special Warfare Center and Commanding Officer, Naval Base Coronado prior to any Raven Operations at Silver Strand Training Complex (SSTC) coordination will take place. Flights will be IAW guidance in an FAA approved COA and the MOU – all flights will remain at or below 400 feet AGL (424 MSL) and remain within the confines of the base property. The Raven UAS is a hand-launched system, launch and recovery procedures are listed below. |  |
| A. Launch Procedure: |  |
| 1. Aircraft and Ground Control Unit (GCU) are assembled |  |
| 2. Aircraft battery is inserted and aircraft establishes GPS link and control link with GCU |  |
| 3. Preflight checks are made to ensure proper operation of aircraft and GCU |  |
| 4. Mission controller holds aircraft in a safe direction pointed into the wind |  |
| 5. Pilot throttles electric motor to 100% |  |
| 6. Battery power is checked |  |
| 7. Pilot gives mission controller green light |  |
| 8. Mission controller throws (hand-launched) aircraft into wind and aircraft achieves flight |  |
| B. Recovery Procedure: |  |
| 1. Aircraft is flown back to GCU site |  |
| 2. Altitude is lowered to a safe height |  |
| a. Safe altitude determined by ground obstructions |  |
| b. Landing altitude typically 50 - 400ft AGL |  |
| 3. When aircraft is at landing location/position, UA is put into “Autoland” mode |  |
| a. Power to motor is cut |  |
| b. Horizontal stabilizer deflects fully upward |  |
| c. Aircraft pitches nose up and begins a steep decent to the ground with wings level (typically a 1:1 glide ratio) |  |
| 4. Aircraft impacts ground causing components to separate (i.e. wings, horizontal stabilizer, etc) and absorb the impact |  |
| 5. Aircraft components are gathered up and inspected for damage |  |
| 6. Aircraft is reassembled |  |