# Emergency procedures

### Low voltage while airborne

If voltage drops to approximately 10.7 Volts, voltage begins to drop rapidly, or there is a noticeable loss of power, then it is advisable to land as soon as possible.

- Trajectory mode – STOP

- Land

- WOW mode – GROUND

### Loss of joystick

A failure of the joystick or communication with the joystick can prevent it from effectively maneuvering the aircraft – in particular a normal landing is not possible.  First, attempt to correct the joystick input.  If neither of these are effective, then it also possible to land with the autopilot without the joystick utilizing these steps.

- Confirm Trajectory mode (“Trj STOP”)

- Confirm this is not actually a loss of data link

- Joystick – reset by unplugging and plugging back in to laptop

- Use the “trajMove land” command to execute a landing approach

- As the aircraft nears the ground type “rc navout.wow=1” into the console, and press enter to go to “ground” mode just as the aircraft touches down

### Loss of GPS

If the GPS unit fails or loses lock (“GPS OFF” or “GPS Nolock”), then the autopilot will only be able to operate for a very short period due to drift in the inertial navigation system.  The fastest method to land should be used.

- Trajectory mode – STOP

- Land immediately (desired <5 seconds)

- WOW mode – GROUND

### Loss of magnetometer (Compass)

If the magnetometer unit fails in flight (“Mag OFF”), then the flight can proceed normally.  The magnetometer is only required prior to takeoff.  If it fails prior to takeoff, then abort the flight.