# Description of Aircraft System

The Hornet Micro Unmanned Aircraft System (UAS) from Adaptive Flight, Inc. is a surveillance system designed to support close-range close-quarters military and civilian reconnaissance missions, Figure 1.  Based on a small helicopter airframe and instrumented with Adaptive Flight’s FCS20 Flight Control System, the 1 kg Hornet Micro UAS is a highly maneuverable and easy to operate aerial image sensor.  Its low weight and electric propulsion make it uncommonly quiet and safe.  The ability to hover and its real time imagery enable operations in close proximity to obstacles and objects of interest.

Figure 1.1, Hornet Micro Unmanned Aircraft, In Flight

Onboard avionics provide the ability for the Hornet to fly in autonomous modes, and to be flown by an operator not skilled or familiar with helicopter flight controls.  These onboard avionics navigate utilizing GPS, inertial sensors (accelerometers and gyroscopes), a magnetometer (compass), and air pressure measurement.  A powerful onboard computer performs the necessary calculations.  A data link with the control station allows the vehicle to report its state to the operator, and for the operator to provide commands.  A fixed camera on the aircraft captures video that is also sent to the operator.  This video is utilized for obstacle avoidance and for performing surveillance missions.

A diagram of signal flow is shown in Figure 2.

tail rotor

video

Tx

Rx

data

Tx

/Rx

FCS20

camera

servos

GPS

Rx

Mag

joystick controller

control station laptop

video

Rx

data

Tx

/Rx

main rotor

control station

Aircraft

Figure 2, Interconnection of major components of Hornet Micro UAS

## Airframe

The airframe is based on the Align T-Rex 450 helicopter, with many currently in use as a model aircraft around the world.  It has a two-bladed main rotor with flybar, and a two-bladed tail rotor.  The skid-type landing gear is retained.  Flight control is accomplished with 4 servos (3 for main rotor, 1 for the tail rotor), a (separate from the other onboard avionics) yaw gyro, and motor speed control.

The avionics are generally mounted on the bottom (flight control systems and video camera).  The video antenna extends out the front.  The data link antenna out the tail.  The GPS antenna and magnetometer (compass) are mounted on the tail.

## Propulsion

Battery has a capacity of approximately 4000 mAh, 11.1V, and weighs approximately 250 g.  A single brushless motor with an approximate maximum power output of 350 W is utilized, which weighs approximately 70 g.

## Onboard Avionics

### Flight Control System

The Hornet Micro UAS is equipped with the FCS20 High-performance UAS autopilot from Adaptive Flight, Inc.  The FCS20 enables advanced behavior in small UAVs.  On the Hornet Micro, it is located under the aircraft, between the landing skids, along with the camera, in a vibration-isolated component.

The FCS20 consists of two credit card sized boards. The processor board employs a Floating Point Digital Signal Processor (DSP) and a Field Programmable Gate Array (FPGA) for optimal performance and design flexibility.  The FCS20 has enough processing capability to enable the most advanced helicopter flight control, along with all other required functions plus room to grow.

- Texas Instrument 300MHz DSP

- Altera Stratix II FPGA

- 256Mb SDRAM, 1Gb Flash

- 3 Rate Gyros and 4 Accelerometers

- GPS receiver module with 4Hz update rate

- Barometric altimeter

- 4 x RS232, 80 Gen. Purpose IO pins

- Size: 55 mm x 85 mm x 28mm

- Weight: 65 g

- (on a separate board) 3-axis magnetometer

### Video System

A small camera is mounted in front of vibration-isolated avionics module.  The centerline is 15 degrees below the longitudinal axis of the aircraft, and it has a 78 degree wide field of view.

Video transmitter/receiver: 1W, 2.4 GHz, 8 channel, tested to approximately 1 Km, actual ultimate range TBD.

### Data Communications

A 900 MHz frequency-hopping, spread-spectrum wireless serial radio is utilized for the data link.  This enables uplink of commands from the control station, and downlink of vehicle status information – including the location of the camera/video.  Aircraft can continue to execute flight plans during loss of link.

- Frequency Hopping Spread Spectrum, 902-928 MHz, 10 hop sequences over 50 frequencies

- Power output, 1000 mW uplink, 500 mW downlink

- Estimated indoor/urban range: up to 3000 feet (900 m)

- Estimated outdoor line-of-sight range: up to 40 miles (64 km)