# Lost Link/Mission Procedures

### General philosophy

The aircraft can be flown by giving the aircraft a flight plan or through joystick control.  The way flight plans are specified it is not possible to have the aircraft “fly away” (that is, keep flying on a particular heading for an undetermined distance).  The way they are constructed, all flight plans must end in a hover or a repeated pattern.  If under joystick control when the link is lost, then the aircraft will stop and hover after a brief time-out (2 seconds).  If a lost link procedure was set up by the pilot, then this safe flight plan will be executed after the selected time out period (e.g. 30 seconds).  The aircraft will then hover at this selected location until battery starvation results in a gradual controlled descent to the surface.

For the proposed operations, the lost link procedure will not be used.  As a result, a lost data uplink will, after 2 seconds, result in a hover at the current location.  The aircraft will hover at this location until battery starvation results in gradual controlled descent to the surface.

### Loss of video link

If video link drops out completely, then corrective action is needed.  First, one should attempt to regain the video link.  If that is not successful, then the landing should be done without it.

- Verify all video connections are connected and receiving power on the ground

- Reset video application on laptop

- Move aircraft closer to control station by observing externally, possibly utilizing an observer to assist

- Land by observing externally (using autopilot or manual safety pilot), possibly utilizing an observer to assist

- WOW mode – GROUND

### Loss of data link

Most panel indications will be crossed out, indicating loss of data link.  Typically, both uplink and downlink will be lost.  One should attempt to regain the link.  If enabled, a lost link flight plan will be executed after the selected period of time (e.g. fly to desired location after waiting 30 seconds).  Move or protect persons or property directly under the aircraft.  If not successful in regaining the link, the aircraft will hover until battery starvation results in a gradual controlled descent to the surface.

- Verify all data connections are connected and receiving power on the ground

- If lost link flight plan is enabled, anticipate that the aircraft will fly to the safe waypoint

- Move or protect persons or property under the hovering aircraft

- Attempt to regain data link by relocating the control station