# Launch/Recovery Description

Launch and recovery is as a helicopter, vertical takeoff and landing.

### Prior to arming engine

Aircraft

- Place aircraft at takeoff location

- Avionics power – ON

Control station

- Turn ON radios (data link and video receiver) and laptop

- Start ground control station application

- Press the “init” button on the console (left most).

- Press the run button on the console (third button, shaped like a triangle pointing to the right).

- Ensure data link established

- Confirm joystick input (on browser window, go from root directory into the “esim” directory.  From there go into the “input” directory.  When a joystick is moved, the numbers on the browser should change).

- Set lost link procedure

- Set waypoints if desired.

- Wait until confirmation that GPS has lock, at least 6 satellites tracked, and the navigation is in “run” mode.

- Confirm navigation solution is reasonable on moving map, including heading.

- Ensure proper loading/calibration of desired map overlays

### Prior to takeoff

Aircraft

- Turn ON servos/motor/video

Control station

- Start video application and confirm video

- Check panel, all green:

  - Confirm data link is good, “Com LINK1”

  - Confirm navigation system is running, “Nav RUN”

  - Confirm GPS is good, “GPS Single” and at least 6 satellites

  - Confirm WOW mode is “wow GND”.

  - Confirm autopilot is off “Aut ON(ni)”

  - Confirm magnetometer is on, “Mag ON”

  - Confirm battery is good, and check voltage

  - Confirm flight plan uploaded

  - Confirm trajectory mode is STOP, “Trj STOP”

### Normal takeoff

Control station

- Press rotor start button on joystick with right thumb

- Wait 15 seconds for rotors to spin up completely (reach a steady-state)

- Move the joystick altitude control to the maximum climb position, left stick up

- Press the takeoff button with right index finger

- Release both joystick after climbing at least 5-15 off the ground, and confirm a stable hover results

NOTE:  While still on the ground, the rotor stop button can be pressed at any time during the spin up procedure to abort the takeoff process.

### Normal Landing

Control station

- Fly the aircraft to a point a few feet above the chosen landing location and achieve a stable hover

- If possible, the remaining phase of landing should be done looking directly at the helicopter, rather than relying on the onboard video – allow time for eyes to adjust

- Press AND HOLD the landing button with right index finger

- Move the joystick altitude control to achieve a slow descent

- Just as the ground is contacted, and while still holding the landing button, press the rotor stop button right thumb.

- Confirm that rotor is spinning down, and WOW mode is “ground”.

- Check final battery voltage

### Shutdown

Aircraft

- (after rotors stopped) Turn OFF servos/motor/video

- Turn OFF avionics

- Charge battery

Control station

- Press the “exit” button on the console

- Turn OFF radios

- Shutdown laptop