# Description of Control Station

The control station consists of the video receiver, data link modem transmitter/receiver, and a ruggedized laptop, Figure 1.  On the laptop, four windows appear, Figure 1.  The upper left window is referred to in this manual as the console.  It is where text can be typed in to provide low-level commands to the system (normally only during setup).  It also has a row of “buttons” that are utilized for a number of commonly used commands.  The lower left window is the referred to here as the browser and can be utilized to examine and modify parameters (also normally only during setup).  The two windows on the right are both scene windows, utilized for graphical display of information, and for inputs utilizing primarily a cursor control device such as a mouse.  An additional window is also used show the onboard video (lower left).

Figure 1, Control station (left) laptop screen, in simulation (right)
(note: lower left window would be replaced by the actual onboard video during flight operations)

## Joystick Input

The primary means of directing flight of the Hornet Micro UAS is the joystick controller input device, Figure 2, a Saitek PC analog gamepad or similar.  The left joystick is utilized to control commanded altitude and heading.  The right stick controls commanded horizontal position.  These commanded paths are ground relative, and so releasing the joysticks will result in a constant commanded position and a stable hover.

Figure 2, Joystick input device functions

Normal takeoff procedure includes first pressing the start rotor button, waiting for rotor spin up, and then pressing the WOW mode “flight” button with the altitude (left stick) in the fully “up” direction.

Normal flight is done with the left and right joysticks only.  Dash mode, when pressed and held, results in 25 ft/sec along the nose of the aircraft added to the commanded velocity.  The joysticks still work, with the velocity being added to this 25 ft/sec.  The heading inputs will effectively turn the aircraft while in dash mode.  Moving the right joystick in the fully “backward” direction will result in a zero commanded speed while in dash mode.

Flight plans can be entered and executed with the aircraft in flight, discussed further below.

Normal landing is performed by flying the aircraft to a point above the intended landing site, then pressing and holding the WOW mode “ground” button, while descending gently to the ground.  At ground contact, press the stop rotor button, which (along with the held WOW mode “ground” button) will stop the rotor, reverse the pitch of the main rotor blades, and stop any further attempts to control the attitude of the aircraft.

## The Console Window

The console consists of a number of buttons for commonly used functions, and a text window for generic text input, Figure 3.

Figure 3, Console window

The buttons and text input area are described here.

### Init button

The normal first action after starting the simulator or control station is to press the “init” button.  This initializes the simulator, including opening the appropriate serial port.  For the control station, pressing it again has no purpose (and should be avoided).  For the simulator, this button can be utilized to restart.

### Pause button

The second button on the console is the pause button.  This is utilized to stop execution of the system.  For the simulator, this effectively pauses the vehicle.  For the control station, this stops rendering of the scene generators and stops checking of the data link.

### Run button

The third button (the right facing triangle) starts (or re-starts after pause) normal execution of the simulator or control station.

### Step button

Only useful in the simulator, this steps the simulator through a small increment of time.

### Fast run button

The fifth button, the double right facing triangles is the run fast button.  Only useful in the simulator, this causes the simulator to run as fast as possible and still update the scene generator once per simulated second.  On a fast machine, this can be 10x real time speed.

### Browser window button

The “brwin” button can be utilized to open additional browser windows, up to a maximum of 3.

### Plot window button

The “plwin” button can be utilized to open plot windows, up to a maximum of 3.

### Exit button

The “exit” button quits the simulator completely.

### Scene window button

The “scene” button can be utilized to open additional scene windows, up to a maximum of 3.

### Trajectory upload button

This “trupld” button uploads the commanded trajectory (waypoints) to the aircraft.  This immediately replaces the trajectory that was being flown.

NOTE:  Uploading waypoints while a currently uploaded set of waypoints is being executed will cause it to switch immediately to the new waypoints, without changing the index of the current waypoint being executed.  Care should be taken when doing this, and the use of this feature should be limited to making minor edits to future waypoints.

NOTE:  If the aircraft is currently not executing a set of waypoints, then the next waypoint to be executed will be reset to the first waypoint if the “trupld” button is pressed.

### Trajectory stop button

The “trstop” button suspends execution of a flight plan or commanded trajectory and places the vehicle in its selected holding pattern (a right-hand circle for airplanes, hover for a helicopter).

### Get directory button

The “getdir” button performs a special function for the browser window.  Pressing this button will result in a data request to the onboard computer to bring down all data in the directory currently being displayed in the first browser window.  This is useful for verifying status/state of many functions.  Not all directories are enabled for this.  The down-linked data is immediately displayed in the browser window.  If the directory could not be requested or was corrupted, then an error message is displayed.

## Console text entry area

The console text entry area allows many text commands to be executed.  For a list of all commands type the command “help”.  It may be necessary to resize the window and utilize the “page up” and “page down” keyboard keys to see all of them.  Only a small fraction are needed for normal operations.  These are:

### “navInit” command

This commands the onboard computer to initiate a recalibration of the navigation system (while on the ground).  The heading is reset to the compass heading.  Position is found with GPS.  Velocity is assumed zero.  Pitch and roll come from accelerometer data.  The initialization stays on hold until at least 6 satellites are being tracked with GPS.  It takes 5 seconds by default (in addition to the time it takes to begin tracking 6 satellites).  When the FCS20 is first turned on, it is in this initialization state.  It will automatically calibrate itself in this way once it has 6 satellites.  As a result, it is normally not necessary to utilize this command.  A typical situation in which it is required is if someone moves the aircraft during the 5 second calibration step, and thus causes a poor calibration.  This is recognized by erratic behavior of the navigation solution showing up on the control station.  If this occurs, the navInit command should be utilized prior to takeoff.

### “trajUpload” command

This command has the same function as the trajectory upload button described above.

### “trajGo” command

This command initiates execution of the currently loaded trajectory, assuming the vehicle is currently not already executing a trajectory (otherwise it does nothing).  The system will attempt to detect if the commanded trajectory has been modified by not yet uploaded, to ensure that the upload is done before execution starts.  If it detects this error, then the message “gcs: need to upload first” appears in the console and the plan execution is not started.  Normal sequence of events:

trajUpload		(upload plan)

trajGo			(begin execution of plan)

trajGo <number>	(begin execution of plan at waypoint <number>)

NOTE:  To prevent accidental engagement, there is no trajGo button.  It must either be typed or done with the menu system on the scene window described below.

### “trajStop” command

This command has the same function as the trajectory stop button described above.

### “trajSave <file name>” command

Once a flight plan has been created, it can be saved to the hard drive with this command.  The plan is stored as an input file (with an .inp file extension).  Up to 20 waypoints can be stored.  To load the flight plan again in the future, the generated flight plan input file should be run.

trajSave myplan		(saves the plan, creates myplan.inp)

@myplan			(loads the plan, destroys any existing plan)

### “trajMove <command>” command

This command is handy for making small controlled changes to the commanded position of the aircraft.

trajMove up 5			(moves the aircraft up 5 feet)

trajMove down 15		(moves the aircraft down 15 feet)

trajMove forward 5		(moves the aircraft forward 5 feet)

trajMove backward 15	(moves the aircraft backward 15 feet)

trajMove left 5			(moves the aircraft to the left 5 feet)

trajMove right 15		(moves the aircraft to the right 15 feet)

trajMove turn 45		(yaws the aircraft to the right 45 degrees)

trajMove turn -90		(yaws the aircraft to the left 90 degrees)

trajMove heading 90		(yaws to a heading of 90 degrees, East)

trajMove land    		(placed aircraft in gradual descent)

trajMove			(reminder of these commands displayed)

NOTE:  The trajMove command creates a new flight plan to perform this function, and so replaces any existing plan.  Save the flight plan prior to executing the command if it will be needed later.

## The Browser Windows

The browser window can be utilized to view and change many parameters.  The data is organized in a “tree” of directories.  Double clicking on any directory leads down that branch of the tree.  Pressing the “up” button in the upper left corner brings one back up the tree, eventually back to the “root”.  The currently active directory is the title of the window, “root” in the case of the example on Figure 4

Figure 4, Browser window

For example, double click on “onboard” to get to data related to the onboard processor.  Next, double click on “actuators” to get to data related to actuators.   Next, double click on “work” to see selected raw data relating to the actuators.  If running the control station with flight vehicle, then press “getdir” on the console a few times to see the values change.  If running the sim, then the values should be updating in real time.  The values “c_delm[0] through c_delf[0]” are the raw actuator commands from the autopilot (whether it’s in control or not).

In actual flight, the “getdir” button on the console can be pressed (for the first browser window) to request this data from the onboard avionics and to display it here.

The window can be closed by pressing the “escape” key on the keyboard while the window is active.

## The Scene Windows

The scene windows contain a broad array of functions and options, the key ones listed here.

Figure 5, Two examples of scene windows: 
(top) primary flight display (bottom) map display

The basic elements of the scene windows, Figure 5, are:

1) The aircraft itself is shown graphically as a helicopter, slightly larger than actual size, in the correct attitude and position based on navigation information coming over the data link.

2) The current commanded position of the vehicle is shown as a yellow circle.  If the autopilot is off, then the commanded position will be at the vehicle.  The aircraft should remain approximately at the center of the circle.

3) The attitude of the view (or horizontal position in the map view) can be changed by holding down the left mouse button and dragging to desired view.

4) Dragging with the left mouse button down on the right edge of the window changes the zoom level.

5) The magenta line extending out of the aircraft shows the flight path if the current trajectory/plan was executed (as the aircraft moves, the entry path will constantly be changing).  If the flight plan is being executed, then the vehicle should follow this path.

6) A single left click in the scene window will cause the coordinates of the intersection of the click with the ground to show up in the console.

7) The black circle in the scene shows the origin or “datum” of the system, all Cartesian coordinates are relative to this location.

### Instrument text display box

The gray box in the upper left corner of the scene window (if active) shows some key numerical information.  For example, the figure above shows speed in feet/second, in this case zero (“0 FPS”), altitude in feet above the initialized location of the navigation system (“29 AGL”), and heading of West (“270Hdg”).  This one is also configured to show the number of GPS satellites tracked if that number is less than 8.  For the Hornet Micro, this also normally shows distance (in nautical miles) “slash” bearing (in degrees) to the aircraft from the datum (“0.04/150”).  If the range is small enough that the bearing is suspect, then two dots appear instead of a numerical value for the bearing.  The final value listed is battery voltage (“11.8 V”).

### Panel

The colored boxes with text in the lower center of the scene window (if active) shows key component status information.  They are color coded, so that normal operation is a dark green that is almost invisible (i.e., in normal state).  Bright green is an alternate “normal” status.  Yellow and red indicate items that require attention.  Each box contains two text elements, the first is the component label, the second is the status:

Table 1, Panel element description

| Label | Description |
| --- | --- |
| Com | Status of communication system: If the data link (down) is out, then many of the other panel elements will be crossed-out, because the status is unknown. If only the uplink is out, then that will be indicated here. |
| Nav | Status of the navigation system, either running or initializing |
| GPS | Status of the GPS, either lock, no lock, or off |
| wow | In the air or on the ground mode |
| Aut | Autopilot status, either “on” or “EXT” (automatic control vs. external/ manual control) |
| Frm | Frame drops shows if computer is being overloaded |
| Mag | Status of magnetometer (compass) |
| DR | Status of high speed data recording (not currently supported) |
| Bat | Status of battery if being monitored |
| Upl | Status of trajectory uploading, either “needed” or “ok” |
| Trj | Status of trajectory/plan execution, either “stop” or “go”, that is, either stopped or going, “stop” means the trajectory execution is off, and joystick control is available |

### Menu Elements

The upper right area contains menus that can be accessed with a cursor control device.  The functions are summarized here.

The menu furthest to the right is utilized to change the type of view, with the name of the current view shown in the window.  “chase” view provides a chase plane style view.  “map” view provides a 2D or 3D navigation view, which is the typical view for editing waypoints.  Map view also adds a 2D/3D toggle menu item (3D mode is useful to visualize vertical profile) and a fixed/center menu item (map stays still or moves with selected object).  “cockpit” provides a simulated cockpit view.

The “show…” menu is utilized to turn on and off various scene generator features.  “text” is the instrument text box discussed above, “HUD” is the heads up display, “vel” shows the velocity vector graphically, “NED” shows the North, East, and Down directions graphically, “body” shows the body axes, “traj” shows the past trajectory the vehicle has flown, “reset” resets the stored past trajectory (useful if one has been flying for a while), “grid” shows a grid with a default 50 ft spacing on the ground, “FOV” shows camera field of view, “texture” shows ground texture maps, and “panel” shows the panel discussed above.

The “plan…” menu is utilized for waypoint/plan editing.  “next >” and “< prev” are utilized to move through the plan to select a single waypoint.  “select” mode can be utilized to click on multiple waypoints to select any subset (holding down the shift key can also be used to select multiple waypoints).  “insert” toggles insert-mode in which any subsequent left mouse clicks on map create new waypoints as a clone of any single selected waypoint.  “delete” deletes selected waypoint(s).  “at veh” makes the selected waypoint at the vehicle.  “save” saves the current plan to “temp.inp”.  “recall” loads “temp.inp”.  “upload” performs a “trajUpload” command discussed previously.  “GO” performs a “trajGo” command as discussed previously.  “STOP” performs a “trajStop” command as discussed previously (“GO” and “STOP” share the same slot).  “GO TO” is similar to “GO”, but sends the aircraft to the currently selected waypoint.

Four more menus show up if a waypoint is selected, they are utilized for waypoint editing.  The “head…” menu is utilized for heading selection.  The “beta 0” choice makes the heading along the direction of flight.  “North”, “East”, “South”, and “West” make the commanded heading mode absolute (rather than relative to the direction of flight) and sets it accordingly.  “cw 10” changes the heading clockwise 10 degrees, and “ccw 10” goes counter clock wise 10 degrees.  Note that this will be relative to the direction of flight, or absolute, depending on whether one started with “beta 0” for relative to the direction of flight (to, for example, fly backwards), or with a “North/East/South/West” choice for absolute.

The “alt…” menu is utilized to change selected waypoint altitude.   The first few choices are height above the datum in feet.  “up 5” and “down 5” increase and decrease by 5 accordingly.  “at veh” makes the commanded altitude the current altitude of the aircraft.

The “vel…” menu is utilized to change selected waypoint speed and acceleration (bank).  The first few choices are absolute speed in feet/second.  The “+5” and “-5” will increment and decrement the speed accordingly.  “acc+1” and “acc-1” increase and decrease the acceleration respectively.

The “type…” menu is utilized to change waypoint type.  The following are the available types of waypoints:

Table 2, Description of waypoint types

| Type | Description |
| --- | --- |
| “cut” | Fly to point and cut corner and proceed on course to the next waypoint |
| “through” | Fly to the point and fly right through it before proceeding on course to the next waypoint |
| “stop at” | Fly to point and stop on it before accelerating to the next point, effectively creating a zero radius turn |
| “E-turn” | Aggressive turn that allows exchange of speed for altitude to create a zero radius turn |
| “landing” | Not supported for the Hornet; Go into a gentle descent – normally not utilized for the hornet because one must use the joystick to change the WOW mode to ground anyway, see normal checklist for performing a landing with the autopilot |
| “chase” | Not supported for the Hornet |
| “formatn” | Not supported for the Hornet |
| “roll” | Not supported for the Hornet |
| “takeoff” | Not supported for the Hornet, see normal checklist for performing a takeoff with the autopilot |
| “<repeat>” | Go back to the first waypoint |

Horizontal position changes are made by clicking and dragging the waypoint(s) or by a left click with the shift key held down.

### Selected waypoint data

If there are selected waypoints/maneuvers, then information is shown about them in the lower left on the scene window.  This information format is as follows:

0 (-300,1000)h=100 v10a5(9) cut p0

The first number “0” is the waypoint number (also shown in the main scene display), a number from 0 to 19 in consecutive order.

The pair of numbers in parentheses “(-300,1000) are horizontal Cartesian coordinates compared to the datum in feet, North/East.  It is normally not necessary to be concerned with these values.

The “h=100” indicates the altitude above the datum (start location) for this waypoint in feet.

The “v10” indicates the speed of the waypoint in selected speed units (consistent with units shown in the instrument text for speed), normally feet.

The “a5(9)” indicates the aggressiveness of maneuver transition.  The “5” is acceleration if feet per second squared (32 is about 1 gravity).  The “(9)” indicates that this aggressiveness level corresponds to 9 degree banked turns.

The “cut” indicates the waypoint type.  In this case, to cut the corner.

The “p0” indicates an absolute heading command of 0 degrees (North, a psi angle of 0 degrees).  Nothing indicates point along direction of flight.  A display of “b180” would be to fly backwards (a beta angle of 180 degrees).