##### Falcon Unmanned Aircraft

##### Description/Technical Specifications

## The Falcon is a hand/bungee launched, remotely operated, unmanned aircraft designed to carry a modular payload such as a gimbaled daylight or IR video camera.

## Falcon Dimensions

- Assembled Dimensions

  - Wing Span: 78 in

  - Height: 12 in

  - Length: 48in

## Weight & Payload

- Empty Weight: 7lb

- Payload Weight: Up to 2.0 lb

- Maximum Gross Take-Off Weight: 9lb

## Materials

- Carbon Fiber (Fuselage, Fuselage Attachment Brackets)

- High Strength Aluminum Alloy (Fuselage Tubes / Internal Wing Structure)

- Fiberglass (Airfoil Surfaces)

- Foam (Airfoil Cores)

- Lightweight Aircraft Plywood (Airfoil End Caps)

- High Impact ABS Plastic (Wing Attach Joint, Servo Control Arms, Bulkheads)

- Aluminum & Stainless Steel Fasteners

- RoHS Compliant

## Propulsion

- Brushless Electric Motor (Tractor/Puller Configuration)

- Folding Fiberglass Impregnated Plastic Injected Molded Propeller

- Electronic Speed Controller

## Power

- Rechargeable Lithium Polymer Batteries

  - 3Cell (11.1V Nominal Voltage)

  - 10Amp-Hr

## Lighting

- High Intensity LEDs

  - Red – Left Fuselage Mount

  - Green – Right Fuselage Mount

  - White – Empennage Mount

  - IR – Top Fuselage Mount

- Purpose: Orientation / Situational Awareness

- Visible Condition Range: Full Darkness to Direct Sunlight

## Operating Requirements

- Operating Temperature: 14° to 104°F (-10° to 40°C)

- Relative Humidity: 0% to 90% Noncondensing

- Maximum Windspeed: 20 knots

## Flight Characteristics:

- Climb Rate: 150ft/min

- Descent Rate: 300ft/min

- Turn Rate: 90°/ 5 sec

- Cruise Speed: 28 knots

- Maximum Speed: 40 knots

- Minimum Speed: 18knots

- Launch Type: Hand Launch or Bungee Launch

- Maximum Altitude ASL: 10,000ft

- Maximum Flight Time: 75 minutes

## 11 Onboard Sensors

- 3 Temperature Compensated Solid State MEMS (Micro-Electro-Mechanical Systems) Gyros

- 3 Temperature Compensated Solid State MEMS (Micro-Electro-Mechanical Systems) Accelerometers

- 1 3-axis Magnetometer (Magnetoresistive Sensors)

- 1 Barometric Pressure Sensor

- 1 Differential Pressure Sensor

- 1 GPS Receiver

## GPS

- GPS Used For: Position, heading, and groundspeed for waypoint Navigation

- Maximum Satellites Tracked Simultaneously: 16

- Position Update Rate: 2 Hz

- GPS Antenna: Ceramic Patch

## Black Box Data Recorder

- Flight Data Recording: Telemetry downlink to ground station at 4 Hz

- Stored To:  Flight log file on ground station hard drive

- Data Recorded: Full flight telemetry including: Airpspeed, altitude, heading, attitude, position, battery voltage, comms health data, GPS data, flight modes, control states, actuator commands, and safety pilot RC inputs.

## RF Communications

- XXXXXXXXXX  Exemption 7 (a) and 7 (e)Data Link

  - Link Type: Vehicle to Ground & Ground to Vehicle (Two-Way)

  - Onboard Antenna: Omni-Directional

  - Controller Antenna: Omni-Directional

  - Transmission Power: 500mW (+20dBm)

  - RF Data Rate: 345 kbps

  - Receiver sensitivity: -108 dBm

  - Transmission Technique: Frequency Hopping/DTS

  - Frequency band: XXXXXXXXXX  Exemption 7 (a) and 7 (e)

  - Certifications: FCC Part 15.247, IC RSS210

  - Data Link Channel Selection: XXXXXXXXX  Exemption 7 (a) and 7 (e)

- XXXXXXXXXX  Exemption 7 (a) and 7 (e)Video Link

  - Link Type: Aircraft to Ground (One-Way)

  - Transmitter Antenna: Omni-Directional

  - Receiver Antennas: Omni-Directional

  - Transmission Power: 1 Watt

  - NTSC Compatible

  - XXXXXXXXXX  Exemption 7 (a) and 7 (e)

## Camera Payloads

- Sony FCB-EX11D Camera on Two Axis Gimbal

  - 530 TV Lines (~640x480)

  - 10x Optical Zoom

  - 12x Digital Zoom

- Dimensions: 35.9 x 40.8 x 59.2 (mm)

  - 2.3”x1.6”x1.4”

  - 0.21 lb

- Tau 640

  - 640x480

  - 1x Optical Zoom

  - 1x Digital Zoom

- Dimensions: 35.9 x 40.8 x 59.2 (mm)

  - 1.4”x1.6”x2.3”

  - 0.21 lb

  - 2.2”x1.5”x1.5”

  - 0.17 lb