MESA COUNTY SHERIFF’S OFFICE

LOST LINK PROCEDURE

FOR

CLMax FALCON

AIR OPERATIONS

The CLMax FALCON has been designed to deal with various fail-safe scenarios such as a lost data link or loss of communications. As pointed out in this section it's one thing to have an aircraft that can deal with this issue and it is also very important to have complimentary crew procedures.

Aircraft Fail Safe procedures:

Before take-off the Pilot in Command (PIC) and observer look over the immediate area or use Google Earth imagery to determine designated "safe zones" for an emergency landing. These areas are agreed upon and other ground crew members involved with the mission are briefed on the location and circumstances for which they will be used.

Aircraft lost data link procedures:

The CLMax FALCON employing the Procerus Kelstral autopilot system was created to allow the PIC and flight crew to proactively manage the threat of Lost Link situations by steadily providing the operator and crew situational data that allows the PIC the option to terminate a mission long before Link is lost.   The Kestral on-board autopilot computer is constantly monitoring the received signal strength and quality of data being exchanged with the PIC Transmitter

In the event of an emergency landing in a safe zone, the Observer will communicate the situation to the ground crew and point out any hazards to the PIC. The Observer will always collocate, within speaking distance, with the PIC mitigating the possibility for loss communications between PIC and Observer.

If the data link fails the PIC will lose communications and subsequent control of the aircraft. If this happens the aircraft autopilot will enter a fail-safe within a predetermined period of the condition being detected and follow one or all of the procedures described below.

User Selectable Failsafe Modes

Stage 1 Modes:

“Fly Home”- The vehicle will navigate to and orbit around its home position, which will be the position at which the UAV was initialized, or the current location of the Commbox if it is equipped with a GPS receiver. The orbit will be established at the “Default Loiter Radius”, and either the “Minimum Home Altitude” or the UAV’s altitude when communications were interrupted, whichever is higher.

“Fly Rally”- The vehicle will navigate to and loiter about its Rally point. The orbit will be established at either the “Minimum Home Altitude” or the UAV’s altitude when communications were interrupted, whichever is higher.

Stage 2 Modes:

“No Land”- The vehicle will continue to orbit at its present position indefinitely. This will eventually result in a spiraling descent to landing once the battery or fuel is expended.

“Land Now”- The vehicle will execute a spiraling descent about its current position to a flared landing immediately when  this failsafe triggers.

“Rally Land”- The vehicle will proceed to its Rally point from its present position. Upon reaching the Rally point, the vehicle will descend to the “Break height” at which time it will depart the Rally point along a user defined heading, descending along a user defined glide slope to the GPS “Land Point”.

Loss of Communication Failsafes

“Loss of Comm During RC Mode”- If the vehicle is in Pilot In Command (PIC) mode and communications with the Ground Control Station are lost for a time equal to or greater than the Stage 1 trigger time, the Stage 1 failsafe will activate. The Stage 1 failsafe will fly the aircraft straight and level from its current position and altitude at the time of the failsafe trigger. If communications have not been re-established by the Stage 2 trigger time, the Stage 2 failsafe will trigger, performing the selected behavior: “Fly Home” or “Fly Rally”.

“Loss of Comm” - If communications with the Ground Control Station are lost for a time equal to or greater than the Stage 1 trigger time, the Stage 1 failsafe will activate. The UAV will then perform the selected behavior: “Fly Home” or “Fly Rally”. If this failsafe is active, it will override behaviors commanded by the “Loss of Comm During RC Mode”. If communications have not been re-established by the Stage 2 trigger time, the Stage 2 failsafe will activate, performing the selected behavior: “No Land”, “Land Now”, or “Rally Land”.

No GPS Lock Failsafe

“No GPS Lock”- If the GPS lock is lost for a time equal to or greater than the Stage 1 trigger time, the Stage 1 failsafe will activate. The UAV will roll into a 20 degree bank and hold its current altitude and airspeed as it attempts to regain a valid GPS signal. If a valid GPS lock has not been re-established by the Stage 2 trigger time, the Stage 2 failsafe will activate, performing a “Land Now” behavior since navigation is not possible without a GPS signal.