Launch/Recovery

Aircraft will be ground transported to the scene of the Certificate of Authorization Training Center at 25° 21’37.88’N, 80°33’48.87W.  The aircraft will be launched and recovered at 25° 21’37.57” N and 80° 33’52.07” W.

The aircraft will be launched, operate within 0.6 nm of the COA center point and recovered at the above launch and recovery point.

UAS flight is only authorized in daylight VFR conditions.

Maximum take off winds are 15 knots.

Aircrew will announce intention to initiate flight over VHF 123.025.

Hover Check:  Immediately after launch PIC will switch to manual mode and bring aircraft into a 6-10 feet hover.  Pilot will command climb, descent, and turn in place to determine aircraft’s responsiveness to commands.  If hover check reveals no control issues the aircrew may proceed to fly the mission.