Miami-Dade Police Department

Micro Air Vehicle

Ground Control Station (GCS) Preparation for Flight: (Pilot in Command):

- Select Start GCS on OCU

- On the OCU, insure that you are on the map screen and select SETUP, then NAVIGATION in order to select MAP SET from drop down menu (for training utilize MDPD Training Set)

- Select OK to close window.

- Select GCS to center map on your location for remainder of setup.

- Select SETUP and then COMM LINK in order to select the proper Avionics pod# (358 or 372)

- Select the LOST LINK RESPONSE to RALLY POINT, then close window by selecting OK.

- Select SETUP and then ALT LIMITS, set up upper altitude limit (200’) and lower altitude (100’) limit

- Wait until AV and GCS indicators on bottom left are green (once AV is turned on)

- If Communication link is established but no video signal then go to Setup, then COMM. LINK and change RADIO CHANNEL until video is received

- Await AV startup to continue

- Select DATA , then PREFLIGHT STATUS and observe OCU as AV completes its preflight and diagnostic checks

- Observe power levels for AV (minimum 70%), OCU (minimum 30%) and GDT (minimum 50%) or confirm that external power is still connected

- When preflight/diagnostic check is complete close window

- Verify with safety officer and/or observer that wind speed is within limits (<15 mph) and the AV is ready for launch

- Verify that the launch area is clear

- Verify that an announcement has been made on 123.02.

- Launch when ready