Aircraft Description:

- Honeywell RQ – 16A Micro Air Vehicle

Air Vehicle is a packpackable ducted fan lift augmented Vertical Take Off and Landing Air Vehicle with gasoline powered engine

Diameter 		23.5 in. with sensor pods

Height 		23 inches

Weight (dry) 		16lbs (EO) 16.3 lbs. (IR)

Fuel Capacity 	2.2 lbs

Flight Duration 	40 minutes

Speed 		45 mph

Climb Rate (max)	10 f/s

Descent Rate (max) 5 f/s

Winds 		15 knots (takeoff/landing), 20 knots (aloft)

Onboard Video  	10 minutes

Battery  		55 minutes (rechargeable)

Launch Capability:  	Manual, Autonomous and Remote

Landing:  		Manual and Autonomous

Flight Control:  	Manual, Autonomous, Semi-Autonomous and Attiude Heading Reference System (AHRS)

Navigation  		SAASM GPS, IMU, Pressure Altimeter and Magnetometer

Communication 	C2 radio and video link radio

Lost Link Response	Ascend , Rally Point, Route Retrace, or Continue

Operating Temp.	0° to 140° F

Storage Temp 	0° to 140° F

Aircraft is equipped with a C2 and video link radio.  Radios can utilize up to 8 selectable channels.  C2 radio is planned to be in the 920 Mhz range and video radio in the 2.4 Ghz range.  Both commercial bands.

Aircraft is equipped with 2 interchangeable payload pods:  a forward and downward looking EO camera or a forward and downward looking IR camera.  The forward camera has a 15°downward pitch.

Electro Optical     	Pixels 768H x 494V   	FOV 46°to 5° (10x optical zoom)

Infra Red		Pixels 320H x 240V		FOV 	40° (4x digital zoom) or

18° (4x digital zoom)