Miami-Dade Police Department

Micro Air Vehicle

Ground Control Station (GCS) Assembly: (Pilot In Command):

- Remove Ground Data Terminal (GDT) and peripherals from storage bag

- Open GDT battery compartment

- Confirm battery is installed or install if necessary

- Close and secure GDT Battery door

- Connect data cable from GDT to OCU (Laptop)

- Connect data link Antennas to GDT (Data link (UHF)) is longer antenna w/ yellow tape.

- Connect video link antenna to GDT (Video is shorter w/Blue Tape)

- Connect power cords to OCU (Yellow Tape) and GDT (Green Tape),if external power is available

- Turn on OCU

- After startup, click on MAV USER to start software

- Select Calibrate Stylus to personalize stylus to user

- Select left or right handed

- Select calibrate stylus and follow on screen prompts

- Select ok to close Window Tablet and Pen Setting screen

- Note numbers on screen to verify Battery is present and charge levels