Emergency Procedures:

The secondary link consists of a 900 MHz modem and a 2.4 GHz modem.  Since the control data is sent through both modems, in the event that one of them losses link, the other will continue to send the control data.  In the event that both of the modems lose link, the pilot will take control of the aircraft using the primary R/C link.

In the extremely rare event of a loss of the primary R/C link, the 2.4 GHz receiver will automatically reduce the power to idle so that the airplane will crash near the position where the link loss occurred.  In the event of catastrophic structural failure, the airplane will crash on the remote R/C flying field over which it is being operated.

In the event of a failure within the MicroPilot autopilot, the pilot will obtain control of the aircraft.  If the pilot is unable to gain control, then the autopilot will execute one of the following failure modes.

- GPS Failure – The autopilot will level the wings and wait for the GPS to reacquire for 10 seconds while maintaining its current course.  Once the 10 seconds has passed it will [use dead reckoning to return to the home position (which will be positioned on the runway)]

- Engine Failure – The autopilot will turn the aircraft towards the runway and descend toward the ground

- 900 MHz and 2.4 GHz modems lost link failure – The autopilot will fly towards and circle around the home position.

- Horizon Communications Link Failure – The autopilot will wait for the communication link to reacquire for 10 seconds while maintaining its current course.  Once the 10 seconds has passed it will return to the home position (which will be positioned on the runway).

- Battery Failure – The autopilot will turn the aircraft towards the runway and descend toward the ground.