PIPER CUB INSPECTION AND

LOG PROCEDURES

Avionics Engineering Center

Ohio University

Athens OH 45701

Table of Contents:

1   Introduction.......................................................................................................................................... 3

2   Inspection............................................................................................................................................. 3

2.1   30 Day Inspection............................................................................................................................. 3

2.2   First Daily Session of Flight Inspection............................................................................................ 3

2.3   Preflight Inspection........................................................................................................................... 4

2.4   Post-Flight Inspection....................................................................................................................... 5

3   Logs..................................................................................................................................................... 5

3.1   Inspection Logs................................................................................................................................. 5

3.2   Flight Logs........................................................................................................................................ 5

3.3   Maintenance Logs............................................................................................................................. 6

3.4   Battery Logs...................................................................................................................................... 6

4   Appendices........................................................................................................................................... 7

4.1   Inspection Checklist.......................................................................................................................... 8

4.2   Flight Checklist................................................................................................................................. 9

4.3   Inspection Log................................................................................................................................. 10

4.4   Flight Log........................................................................................................................................ 11

4.5   Maintenance Log............................................................................................................................. 12

4.6   Battery Log..................................................................................................................................... 13

1   Introduction

The purpose of this document is to provide detailed inspection and log procedures

2   Inspection

The inspection procedures for the Dave Patrick Model’s Piper Cub are as follows.

2.1   30 Day Inspection

The following inspection shall be performed at least 30 days prior to the flight of the aircraft.

a)  All peeling monokote shall be repaired.

b)  All weights used for balance shall be checked to ensure they are secure.

c)  All servo arms shall be checked for screws and a tight fit.

d)  All servo linkages shall be thoroughly checked to ensure they are securely fastened to the 	servo arm and the control surface.

e)  All servo connectors shall be checked to ensure they are fully connected to the receiver.

f)  All screws shall be checked to ensure they are not stripped.

g)  All bolts shall be checked to ensure they are tight.

h)  All surfaces shall be checked for strength.

i)  The wing-halves shall be checked to ensure that they are attached properly.

The inspection checklist is located in appendix 4.1

2.2   First Daily Session of Flight Inspection

The following shall be performed prior to the first session of flight of each day.

a)  The balance of the aircraft shall be checked to ensure that it is within the limits provided by 	the aircraft manual, between 4 and 4 7/8 inches from the front edge of the main wing.

b)  The range of the transmitter shall be checked to ensure safe flight. The pilot shall walk 30-50 	paces away from the aircraft while holding down the F/S Range switch and moving controls.  	c)  The observer shall hold the aircraft and confirm that the controls are moving completely and 	correctly.

d)  The engine shall be checked to ensure that it turns off if the transmitter is out of range.  The pilot shall increase the throttle of the aircraft and turn off the transmitter ensuring that the engine turns off.  The observer shall hold the aircraft during this process.

The first session of flight checklist is located in appendix 4.2

2.3   Preflight Inspection

The following shall be preformed prior to each flight of the aircraft.

a)  All batteries, including but not limited to the receiver battery and the transmitter battery, shall be checked and recorded to ensure they are sufficient to provide the power needed for flight.

b)  The propeller shall be checked to ensure that it is free of chips.

c)  The spinner shall be checked to ensure that it is tight.

d)  The wheels and wheel struts shall be checked to ensure that they are secure.

e)  All control surfaces shall be checked to ensure that they are not separating from the wings, 	the vertical stabilizer, or the horizontal stabilizer.

f)  All servo arms shall be checked to ensure that they have screws.

g)  All servo arms shall be checked to ensure a tight fit by rotating them.

h)  All linkages shall be checked to ensure that they are connected properly without any 	abnormalities.

i)  The engine and mount shall be checked to ensure they are secure.

j)  All control surface and throttle movements shall be checked to ensure they are correct.

The preflight checklist is located in appendix 4.2

2.4   Post-flight Inspection

The following shall be performed after each flight of the aircraft.

a)  All receivers and transmitters shall be checked to ensure they are off.

b)  The remainder of the flight information shall be recorded in the flight log.

c)  All battery information shall be recorded in the appropriate battery logs.

The post-flight checklist is located in appendix 4.2

3   Logs

3.1   Inspection Logs

All inspections shall be recorded in the inspection log.  The following shall be recorded in the inspection log.

a)  An inspection number shall be given to and recorded on the form.

b)  The date of the inspection shall be recorded.

c)  The person conducting the maintenance shall be recorded.

The inspection log is located in appendix 4.3

3.2   Flight Logs

All flights shall be recorded in the flight log.  The following shall be recorded in the flight log.

a)  The name of the aircraft shall be recorded.

b)  The location of the flight shall be recorded.

c)  The name of the pilot shall be recorded.

d)  The name of the observer shall be recorded.

e)  A flight number shall be given to and recorded on the form.

f)  The page number will be recorded on the form.

g)  The date of the flight shall be recorded.

h)  The duration of the flight shall be recorded on the form.

The flight log is located in appendix 4.4

3.3   Maintenance Logs

All maintenance that is performed on the aircraft shall be recorded in the maintenance log.  The following shall be recorded for each part that undergoes maintenance.

a)  The name of the aircraft shall be recorded.

b)  The name of the person conducting the maintenance shall be recorded.

c)  A maintenance number shall be given to and recorded on the form.

d)  The page number shall be recorded on the form.

e)  The date of the completion of the maintenance shall be recorded.

f)  The parts related to the maintenance shall be recorded.

g)  Detailed notes shall be taken of the maintenance that was performed.  When possible, 	pictures shall be attached to the form showing the process of the maintenance.

The maintenance log is located in appendix 4.5

3.4   Battery Logs

Each battery used in any kind of flight operation shall have a battery log.  The following shall be recorded for each battery and flight.

a)  The date the battery was used shall be recorded.

b)  The flight number shall be recorded.

c)  The flight time shall be recorded.

d)  The voltages of each battery contained on board the aircraft shall be recorded prior to and 	after the completion of each flight in each battery log.

e)  The number of mAh put into the battery when charged shall be recorded

The battery log is located in appendix 4.6

4   Appendices

4.1   Inspection Checklist

4.2   Flight Checklist

4.3   Inspection Log

4.4   Flight Log

4.5   Maintenance Log

4.6   Battery Log

4.1   Inspection checklist

Piper Cub Inspection Checklist

___   Repair any peeling monokote

___   Check all ballasts to ensure they are securely fastened to the aircraft

___   Confirm that all servo arms have screws and have a tight fit.

___   Verify that all servo linkages are securely fastened to the servo arm and the control

surface.

___   Check all servo connectors to ensure they are fully connected to the receiver.

___   Check all screws to ensure they are not stripped.

___   Check all bolts to ensure they are tight.

___   Verify that all surfaces are strong

___   Confirm that the wing-halves are attached properly.

4.2   Flight Checklist

Piper Cub Flight Checklists

First Session of Flight

___   Verify the balance of the aircraft is within 4 and 4 7/8 inches from the leading edge of the

wing.

___   Check the range of the transmitter

___   Ensure that the engine turns off if the transmitter is out of range by turning off the 				transmitter

Preflight

___   Check and record all transmitter and aircraft battery voltages

___   Ensure that the propeller is free of chips

___   Confirm that the spinner is tight

___   Verify the wheels and wheel struts are secure

___   Check all the control surfaces to ensure they are not separating from the wings, the

vertical stabilizer, or the horizontal stabilizer

___   Verify that all servo arms have screws and have a tight fit

___   Confirm that all linkages are connected properly

___   Check to ensure that the engine and mount are not loose

___   Verify that all control surface and throttle movements are correct

Post-flight

___   Verify that all receivers and transmitters are off

___   Record all flight information in the flight log

___   Record all battery information in the battery logs

4.3   Inspection Log

Inspection Log

Aircraft:  Piper Cub 1									Page:  ______

| Inspection Number: | Date | Person Conducting Maintenance |
| --- | --- | --- |

4.4   Flight Log

Flight Log

Aircraft:  __________________________					Flight Number:  __________

Location of Flight:  _______________________				Page:  _____

Name of Pilot:  _____________________					Date:  __________

Name of Observer:  _____________________				Flight Time:  ___________

___   First Session of Flight Checklist Complete

___   Preflight Checklists Completed

___   Post-flight Checklist Completed

| Battery Description: | Voltage Prior to Flight | Voltage After Flight |
| --- | --- | --- |

Notes:

4.5   Maintenance Log

Maintenance Log

Aircraft:  ___________________________				Maintenance Number:  __________

Person conducting maintenance:  ______________________	Page:  ______

Date:  ___________

Related Parts:

Notes:

4.6   Battery Log

Battery Log

Battery Description:  ________________________________				Page:  ______

| Date: | Flight Number: | Voltage Prior to Use | Voltage After Use | mAh |
| --- | --- | --- | --- | --- |