Control Station:

Our control station consists of an R/C pilot, a ground station operator, and an observer.  The pilot will be controlling the aircraft using a conventional R/C transmitter/receiver pair.  The base station contains a 2.4 GHz and a 900 MHz modem.  Both of the modems send control data from the PC to the autopilot, but the 900 MHz modem also receives telemetry data.  The pilot will have authority over whether or not he or the autopilot has control of the aircraft through a switch to a redundancy system that we have created that use relays to send control information to the servos.  In the event that the relays in the redundancy system lose power, they will revert to the primary R/C transmitter/receiver pair.

The base station of the MicroPilot MP2028 system is connected to a computer to send control data to the autopilot and to receive telemetry information from the autopilot.  The software used on the computer is MicroPilot’s Horizon Software.  The Horizon software shows the airspeed, the altitude, the heading, the orientation of the aircraft, the battery voltages, the location of the aircraft on a map, and the status of the control, telemetry, and GPS communication links.  Using the software allows us to change the desired airspeed, the desired altitude, and the waypoints on the fly as well.  The software will also notify the user if an error in the autopilot occurs.

The pilot will have at least a class 2 medical and have passed the written exam for the private pilot license.  The pilot will keep the aircraft in their field of view at all times.  The R/C pilot will at all times have an observer who assists the pilot with avoidance of other traffic, obstructions and terrain.  The observer will have at least a class 2 medical.  In some cases, the R/C pilot may also be accompanied by an additional person to help with data recording.  The R/C transmitter that the pilot will be using with the aircraft is a Futaba 9CHPS transmitter (stock and manufacturer number FUTK78).  The module is a Futaba TM-8 8-Channel 2.4GHz FASST Module (manufacturer number FUTL8928).  The receiver is a R6008FS 8-Channel Receiver.