BAT Emergency Procedures

Loss of GPS

Definition

GPS signal is lost for more than 10 seconds

Indicators

- Voice warning “No GPS”

- GPS status on ground station indicates “No Sat” or “No Com”

- Return to home is initiated automatically: aircraft returns to home location using inertial sensors, airspeed and altimeter, flying at 35 knots and 400 AGL along the calculated home heading.

- Moving map display shows aircraft’s changing heading, altitude and speed and other flight states, but not the aircraft’s position.

Actions CONTROL

- Inform crew about loss of GPS incident and subsequent Return to home mode

- Monitor aircraft’s actions and distance to home

- If GPS signal is re-acquired, aircraft will resume waypoint navigation

- If GPS is not re-acquired, communicate with PILOT to take over 					manually when in transmitter range

- Inertial navigation is not as precise as GPS nav, be sure to turn on video 				recorder and see if you can track the aircraft’s position using the 					downlinked video.

Actions PILOT

- Take over manually upon request of CONTROL when aircraft in transmitter range

- Land aircraft manually

Loss of 900 Mhz link

Definition

900 Mhz link is lost for more than 10 seconds

Indicators

- Telemetry wheel on ground station stops turning, and no audible ping, and no telemetry data updates in graphs, dials and gauges

- Return to home is initiated automatically: aircraft returns to home location using inertial sensors, airspeed and altimeter, flying at 35 knots and 400 AGL along the calculated home heading.

Actions CONTROL

- Inform crew about loss of 900 MHz link and subsequent Return to home mode

- Monitor aircraft’s actions and distance to home

- If 900 Mhz link is re-acquired during return to home, select “go to 					waypoint xx” or cycle autopilot switch on RC transmitter to resume 				waypoint navigation

- If 900 MHz link is not re-acquired, communicate with PILOT to take over 				manually when in transmitter range

Actions PILOT

- Take over manually upon request of CONTROL when aircraft in transmitter range

- Land aircraft manually

Engine RPM drops below 1000 RPM

Definition

Aircraft engine RPM drops below 1000 rpm

Indicators

- Potential voice warnings: “Low airspeed”, “Altitude off track”, “Engine has stopped”

- RPM indicator on ground station shows rpm status

- Return to home is initiated automatically: aircraft returns to home location using inertial sensors, airspeed and altimeter, flying at 35 knots and 400 AGL along the calculated home heading. If RPM is not restored, aircraft will glide to home location at 35 knots

- Moving map display shows aircraft’s changing heading, altitude and speed and other flight states.

Actions CONTROL

- Inform crew about engine RPM drop below 1000 and subsequent Return to home mode

- Monitor aircraft’s actions and distance to home

- Communicate with PILOT to take over manually when in transmitter range

Actions PILOT

- Take over manually upon request of CONTROL when aircraft in transmitter range

- Land aircraft manually if feasible

- If manual landing not feasible, kill engine and ditch aircraft

Transmitter (72 Mhz link) failure

Definition

Transmitter does not respond

Indicators

- Controls on transmitter do not function

- Low battery

Condition A. If aircraft was in autonomous mode and transmitter failure occurred while switching from autonomous to manual mode for landing (aircraft remains in autonomous mode):

Actions PILOT

- Inform CONTROL of transmitter failure

- Switch to backup battery in transmitter while maintaining autonomous flight

- Re-establish manual control

- If manual control cannot be established, continue autonomous flight

Actions CONTROL

- Monitor aircraft’s mode (manual, waypoint) on ground station

- If manual control is established, continue manual flight or landing

- If manual control is not established, continue flight in autonomous mode

- Load landing flight plan

- Land aircraft autonomously

Condition B. If aircraft was in manual mode and transmitter failure occurred while in this mode:

In this case, the aircraft switches automatically to autonomous mode

Actions CONTROL

- Inform crew of transmitter failure

- Switch to backup battery in transmitter

- Re-establish manual control or

- Land aircraft autonomously

Dead Engine

Definition

Engine has died

Indicators

- Potential voice warnings: “Low airspeed”, “Altitude off track”, “Engine has stopped”

- RPM indicator on ground station shows rpm status (zero)

- Return to home is initiated automatically: aircraft glides to home location using inertial sensors, airspeed and altimeter

- Moving map display shows aircraft’s changing heading, altitude and speed and other flight states.

Actions CONTROL

- Inform crew about dead engine and subsequent Return to home mode

- Monitor aircraft’s actions and distance to home

- Communicate with PILOT to take over manually when in transmitter range

Actions PILOT

- Take over manually upon request of CONTROL when aircraft in transmitter range and within 5:1 glide from airstrip

- Land aircraft manually if feasible

- If manual landing not feasible, ditch aircraft

Airspace conflict

Definition

Another aircraft is approaching or entering UAV airspace

Actions VISUAL OBSERVERS

- Inform  CONTROL and PILOT of airspace conflict with estimated heading, distance, airspeed

Condition A. UAV aircraft is within RC range

Actions CONTROL

- Communicate with PILOT to take over manually and divert to resolve conflict

- Monitor aircraft’s actions

Actions PILOT

- Take over manually upon request of CONTROL

- Divert aircraft to resolve conflict

Condition B. UAV aircraft is not within RC range

Actions CONTROL

- Select “go to waypoint xx”, choosing waypoint that resolves conflict

- Upload course to aircraft

- Monitor aircraft’s actions

- Return aircraft to home location and determine cause of conflict

Deviation from flight plan

Definition

Aircraft is deviating from programmed flight plan

Indicators

- Moving map display shows aircraft (red delta) has deviated from flight plan: commanded path (green line) is different from actual path flown (blue line). Note: deviations of up to 0.15 miles may occur in high wind conditions and this is normal.

Condition A. UAV aircraft is within RC range

Actions CONTROL

- Inform crew about deviation from flight plan

- Communicate with PILOT to take over manually

- Monitor aircraft’s actions

Actions PILOT

- Take over manually upon request of CONTROL

- Land aircraft manually

Condition B. UAV aircraft is not within RC range

Actions CONTROL

- Inform crew about deviation from flight plan

- Select “go to waypoint xx”, selecting a waypoint that will initiate change in heading

- Monitor aircraft’s actions

- If aircraft responds, resume flight plan

- If aircraft does not respond and continues deviation, initiate flight termination, aircraft will ditch

Computer or software lock-up

Definition

Either the software has crashed or the computer locked up in-flight

Indicators

- MLB Software is frozen, or computer locked up entirely

Condition A. UAV aircraft is within RC range

Actions CONTROL

- Inform crew about system lock-up

- Determine if switch to manual control is needed

- If so, communicate with PILOT to take over manually

- If not, restart MLB software and/or laptop, download flight plan from 	aircraft and resume autonomous flight

Actions PILOT

- Take over manually upon request of CONTROL

- Land aircraft manually if requested by CONTROL

Condition B. UAV aircraft is not within RC range

Actions CONTROL

- Inform crew about system lock-up

- Restart MLB software and/or laptop, download flight plan from aircraft and resume autonomous flight