LAUNCH/RECOVERY  (Not An Autonomous System)

# T-2 Day Checks

1) Visually Inspect:

Propeller and Motor

Wing, Attachment, and Ailerons

Tail and Control Surfaces

Payload Systems (Video Antenna Must be Straight and Pointed Down)

Video System Ground Station

2) Measure Weight and Center of Gravity - Record

3) Preflight Planning Session:

Mission Objectives, Flight Profile, Contingency Plan

Vehicle/Systems Status

GO/NO GO Criteria (wind limits, cloud cover, rain)

Personnel Roles and Responsibilities (Pilot, Payload Operator/Launch Person, other participants)

4) Charge Batteries:

Vehicle (One Lithium Pack)

Control System Transmitters (JR XP662 – RC, & Futaba SS3 - payload)

Video Receiver

Digital 8 VCR

# T-1 Day Checks

5) Confirm Battery Charging Complete:

Vehicle (Single Lithium Pack)

Control System Transmitters

Video Receiver

Digital 8 VCR

6) Confirm Vehicle and Payload Systems OFF

Vehicle Batteries Installed and Unplugged

Vehicle Control System Switch OFF

Payload System Switch OFF

Radio Transmitters OFF

Video Receiver Plugged into VCR and OFF

VCR OFF

7) Wing Attachment:

Slide Wing Halves Together

Fasten Nylon Strap to Hold Wings Together

Connect Port Aileron Linkage

Connect Aileron Cable from Wing to Cable in Fuselage

Attach Wing to Fuselage with Two Nylon Screws

8) Vehicle and Payload Systems Checks:

Video Receiver ON

VCR ON

Constrain Vehicle

Vehicle Control Transmitter ON (JR XP662)

Confirm Throttle 0%

Plug in Vehicle Battery

Confirm “beep” from motor controller

Install Hatch (check that latch is engaged)

Clear Propeller!

Vehicle Control System Power Switch ON

Confirm final arming “beep” from motor controller

Control Checks:

Aileron LEFT

Aileron RIGHT

Elevator UP

Elevator DOWN

Rudder LEFT

Rudder RIGHT

Confirm All Controls Are Functioning Properly

Payload Control Transmitter ON (Futaba SS3)

Payload Control Power Switch ON

Verify Good Image on VCR

START VCR Recording

Switch between Infrared (IR) to Video Cameras

Verify Good Image on VCR

Constrain Vehicle!

Advance Throttle Slowly to 100% for Approximately 3 Seconds

Retard Throttle Slowly to 0%

Confirm All Controls Are Functioning Properly

Verify Good Image on VCR

STOP VCR Recording

Payload Systems Switch OFF

Vehicle Control System Switch OFF

Vehicle Control Transmitter OFF (JR XP662)

Payload Control Transmitter OFF (Futaba SS3)

Video Receiver OFF

VCR OFF

Open Hatch and Unplug Vehicle Battery

9) Recharge Batteries as Required:

Vehicle (for Propulsion, Control and Payload)

Control Transmitter

Video Receiver

Digital 8 VCR

# PreFlight Checks

10) Confirm Battery Charging Complete:

Vehicle (Propulsion, Control and Payload)

Control Transmitter

Video Receiver

Digital 8 VCR

11) GO/NOGO Decision

12) Transport vehicle and support equipment to flight area

13) Record Meteorological Conditions:

Wind Speed and Direction

Temperature and Humidity

Barometric Pressure (optional)

14) Confirm Operations Area is clear of non-participating personnel

15) Visually Inspect:

Propeller and Motor

Wing, Attachment, and Ailerons

Tail and Control Surfaces

Payload Systems (Video Antenna Must be Straight and Pointed Down)

Video System Ground Station

16) Wing Attachment

Slide Wing Halves Together

Fasten Nylon Strap to Hold Wings Together

Connect Port Aileron Linkage

Connect Aileron Cable from Wing to Cable in Fuselage

Attach Wing to Fuselage with Two Nylon Screws

17) Final Vehicle Preparation:

Video Receiver ON

VCR ON

Vehicle Control Transmitter ON (Futaba T6XA)

Fully Extend Vehicle Control Transmitter Antenna

Confirm Throttle 0%

Plug in Vehicle Battery

Install Hatch (insure latch is engaged)

Clear Propeller!

Vehicle Control System Switch ON

Control Checks:

Aileron LEFT

Aileron RIGHT

Elevator UP

Elevator DOWN

Rudder LEFT

Rudder RIGHT

Confirm All Controls Are Functioning Properly

Payload Control Transmitter ON (Futaba SS3)

Fully Extend Vehicle Control Transmitter Antenna

Payload Control Switch ON

Verify Good Image on VCR

START VCR Recording

Switch between Infrared (IR) to Video Cameras

Verify Good Image on VCR

Constrain Vehicle!

Advance Throttle Slowly to 100% for Approximately 3 Seconds

Retard Throttle Slowly to 0%

Confirm All Controls Are Functioning Properly

Verify Good Image on VCR

18) Confirm Launch Direction and Flight Pattern

19) Confirm Pilot is GO

20) Confirm Launch Person is GO

Launch and Flight

21) Launch Person: Confirm Launch Area is Clear, Announce “Clear and Ready”

22) Pilot: Announce “Ready to Launch”

23) Launch Person: Annotate 3-Second Countdown

24) Command Motor ON and Launch Into Wind

25) Climb and Maneuvering Flight

26) Fly Mission Profile as Required

27) Launch Person: Confirm Landing Area is Clear and Announce “Clear to Land”

28) Pilot: Announce “Landing”

29) Begin decent at 400 ft.

30) Slow descent in direction of designated grass landing strip.

31) Descent consists of a downwind leg, crosswind leg and final approach into the wind.

32) Motor off approximately 20 ft AGL and land with skid recovery.

33) RC Receiver OFF

34) RC Transmitter OFF

35) Sensor Systems OFF

36) GSE OFF

37) Safe Vehicle: Open Hatches/Remove Wing, Unplug Propulsion Batteries

Emergency Procedure: for Rapid Flight Termination - a Descending Spiral for an estimated descent rate of ~500 fpm will be commanded

38) SAFE Vehicle and Ground Station:

Payload Systems Switch OFF

Vehicle Control System Switch OFF

Vehicle Control Transmitter OFF (Futaba T6XA)

Retract Vehicle Control Transmitter Antenna

Payload Control Transmitter OFF (Futaba SS3)

Retract Payload Control Transmitter Antenna

Open Hatch and Unplug Vehicle Battery

VCR STOP and OFF

Video Receiver OFF

# Post Flight

39) Visually Inspect:

Propeller and Motor

Wing, Attachment, and Ailerons

Tail and Control Surfaces

Payload Systems

Video System Ground Station

40) Post Flight Review Meeting (Flight Objectives Achieved? Problems? Next Steps?)

41) Complete Flight Log