Lost link/Mission Procedures and Lost Communications Procedures

NOTE: Lost link/Mission Procedures and Lost Communications Procedures were both included in this document.

Loss of Command and Control Link

If the 900 MHz modem link is lost for 10 or more seconds, the aircraft will automatically return to home location. Return to home is performed at 750 ft above launch point altitude and 35 knots.

After the initial startup sequence has been completed, the 900 MHz link is not essential to maintain flight. The aircraft can operate independent of the 	groundstation once a flightplan has been uploaded, so there is no immediate problem if communication is lost or the ground station fails. However, the 2 way link is required to retask the aircraft and receive flight status updates.

Loss of command and control link is recognized by noticing the loss of the 1 Hz ping, indicated visually and audibly on the ground station. The UAS recognizes the loss of the command link by not receiving 1 Hz update rates. The Return to home mode is initiated automatically upon loss of link for 10 or more seconds

Loss of navigation

If GPS signal is lost for more than 10 seconds, the aircraft returns to the home location, using the on-board compass and flying at 35 knots and 750 ft AGL along the calculated home heading. The R/C pilot can then take over manually. If GPS signal is reacquired, the aircraft will resume waypoint navigation to the point it was headed prior to loss of GPS.

Primary navigation loss is indicated on the ground station through the GPS status indicator updated at 1 Hz intervals, visually and audibly with a ping.

Back-up navigation can be provided by switching from autonomous to manual control on the RC control unit within the 1-2 mile radius.

Return home modes

Conditions that cause return home mode are loss of the 900 MHz modem link or loss of GPS signal for 10 or more seconds. Verifying the return to home mode is included on the

preflight checklist, and is also displayed after a flight plan is loaded. Return to home means the aircraft flies directly from point of loss of link to home point. Upon returning to home, operator can switch to manual control of aircraft and land manually if GPS is unavailable or jammed. If GPS is reacquired, a new flight plan can be uploaded.

Loss of RC control only

If RC control is lost during manual operation (such as battery failure in RC unit), a 2nd backup RC transmitter of the same type is available.