- CONTROL STATION

ImageAire iii-2 has 2 separate radio control systems, one for flight control and the other for payload control. Both transmitters are completely license free and require no special clearances to operate.

The flight control system is a commercial PCM radio control system by JR (JR XP662).  Control configuration is standard US format with throttle and rudder on the left stick and elevator and ailerons on the right stick.  Please note that the elevator and aileron dual-rate selector switches on the top-left and top-right of the transmitter are NOT active, but may be programmed at the pilot’s discretion.  The rudder and ailerons are mixed electronically with the transmitter to facilitate smoother turns and enhance low speed performance.  The settings for all controls deflections may be modified at the operator’s discretion if deemed necessary.  See the JR radio manual for instructions in the attached document.

The payload video downlink feed is controlled by a second FM radio control system by Futaba (Futaba SS3).  The downlinked video feed may be switched between the color camera and infrared system by operating the slide switch located on the left-front side of the transmitter.

Either RC transmitter will function for a minimum of 2 hours before needing recharging.  The charge level can be monitored by observing the indicators on the face of the transmitters.