ImageAire iii-2

USDA SUAS Operating Procedures

Updated: January 15, 2008

# T-1 Day

# Note Date:

1) Verify Mission and Systems Configuration

2) Charge Batteries:

Vehicle Control

Vehicle Propulsion

Radio Control (RC) Transmitter

Sensor/Data Systems

3) Measure:

Weight:

Center of Gravity:

4) Preflight Checks:

Sensors

Data Systems

Vehicle Systems

# T-3 Hours

Note Date:

5) Preflight Planning Session:

Mission Objectives Flight Profile, and Contingencies

Vehicle/Systems Status

GO/NO GO Criteria

# T-1 Hour

Note Time:

6) Complete Battery Charging:

Vehicle Control:

Vehicle Propulsion:

RC Transmitter:

Sensor/Data Systems:

7) Record Meteorological Conditions:

Wind Speed and Direction:

Temperature and Humidity:

Barometric Pressure:

8) Preflight Checks:

Sensors

Data Systems

Vehicle Systems

9) GO/NOGO Decision

# T- 30 Minutes

10) Transport vehicle and support equipment to flight area:

Vehicle

RC Transmitter

Video Receiver, Video Recorder, Tape

Project Radio(s)

Wind Gage/Flag

Documentation including MSDS

Fire Extinguisher

11) Confirm Flight Hazard Area is clear of non-participating personnel

12) Set Roadblocks (as required)

13) Communications Equipment Check

Telephone

(Cell Phones OFF)

14) Advise: “Operations Commencing”

15) Inspect:

Propeller and Motor

Wing, Attachment and Ailerons

Tail and Control Surfaces

Sensor Systems

# Preflight, Launch and Flight

Note Time:

16) Final Vehicle Preparation:

Restrain Vehicle

Clear Propeller

Plug in Propulsion Batteries

Secure Hatches/Wing

17) Ground Support Equipment (GSE) ON – Start RECORDING

18) Sensor Systems ON

19) RC Transmitter ON

20) RC Receiver ON

21) Motor ARM (as applicable)

22) Control Check:

Aileron LEFT

Aileron RIGHT

Elevator UP

Elevator DOWN

Rudder LEFT

Rudder RIGHT

Motor ON

Motor OFF

23) Fail-Safe/Interference Check:

Motor SLOW

RC Transmitter OFF

Confirm Fail-Safe (Rudder LEFT, Elevator UP, Motor OFF)

Confirm No Un-Commanded Control Surface Movements

RC Transmitter ON

24) Range Check (as required)

25) Confirm Launch Direction and Flight Pattern

26) Confirm Pilot is GO

27) Confirm Launch Area is Clear and Announce “Launching”

28) Motor ON and Launch (Start Timer)

29) Climb and Maneuvering Flight

30) Research Measurements and Data Acquisition

31) Confirm Landing Area is Clear and Announce “Landing”

32) Land Vehicle (Stop Timer)

33) RC Receiver OFF

34) RC Transmitter OFF

35) Sensor Systems OFF

36) GSE OFF

37) Safe Vehicle: Open Hatches/Remove Wing, Unplug Propulsion Batteries

Emergency Procedure: for Rapid Flight Termination - a Descending Spiral for an estimated descent rate of ~500 fpm will be commanded

# Post Flight

38) Advise “Operations are Complete”

39) Inspect Vehicle

40) Remove Roadblocks

41) Download Sensor Data (as required)

42) Complete Flight Log (attached)

Flight Log

Date:

UAV System:

Mission Objectives:

Wind Speed and Direction:

Temperature:

Humidity:

Barometric Pressure:

Launch Time:

Landing Time:

Total Flight Time:

Mission Results:

Post Flight Status: