Emergency Procedures

Following are some procedures that are in place in case of the following potential emergencies:

Unresponsive Flight Controls

If a servo or flight control sticks, the aircraft will be out of control. The autopilot tries to correct the situation as best as possible with the remaining servos, but there is no way to detect a stuck servo. The operator can execute the flight termination (i.e. engine kill) command if this situation occurs.

Stuck Throttle

If the throttle is stuck, the aircraft will continue to fly normally, except that altitude will not be regulated. There is no sensor to specifically detect this failure, but erratic RPM or poor altitude tracking are the symptoms. The procedure for recovery is to return to base and land or "ditch" the aircraft manually.

Loss of Propulsion

The aircraft automatically glides toward home location at 35 knots. The engine cannot be restarted in flight. Electrical power is not necessarily lost if the engine fails. If aircraft is within the 1-2 mile RC control range, it can be ditched or landed manually.

Loss of Electrical Power

If the power is lost on the ground station, the aircraft still navigates to the waypoints, flying via GPS and uploaded flight plan. Once the laptop and ground station are restarted, the flight plan can be downloaded from the aircraft to the ground station and the link is reestablished. A backup laptop with identical software and flight plans is part of the systems and can be used.  Eight 12V 7 mAh automotive batteries are used to power the systems in the field.

The aircraft has a 14.4 V Lithium-Ion battery with a time life of 5-6 hours. There is a life meter with the voltage indicator on the ground station. The minimum allowed voltage is 13 volts and is monitored during flight. The aircraft will not be launched if the battery voltage <14.5 V. The payload is on separate power circuit and UAS will still fly if payload is jammed and looses power.

Emergency Flight Termination System (FTS)

Emergency FTS for this aircraft would be to stop the engine. This can be accomplished by using the long range (10 mile) 900 MHz communication link in the ground station or using the shorter range (1-2 mile) radio control hand-set. Criteria would be leaving the predetermined flight plan, lack of containment in operating range, or return home failure

If aircraft is below RF horizon, it is likely it lost GPS link, upon which it will return to home. After propulsion is terminated, the aircraft automatically glides home at 35 knots.