- CONTROL STATION

ImageAire Lite ii-2 has one radio control for flight control and video transmitter downlink control.  The transmitter is completely license free and requires no special clearances to operate.

The flight control system is a commercial PCM radio control system by Futaba (6EXAP 72.270 MHZ/Ch. 24). Control configuration will be throttle on the left stick and elevator and rudder controls on the right stick since this UAS has no ailerons.  The settings for all controls deflections may be modified at the operator’s discretion if deemed necessary.

Channel 4 will be designated to toggle the downlink for the color camera’s 2.4 GHz video transmitter.