2.  COMMUNICATIONS SYSTEM

The Communications System consists of three components: 1) Imaging Payload, 2) Video Systems Ground Station, and 3) Pilot – Observer verbal communication.

All Components are described in detail below:

IMAGING PAYLOAD SYSTEMS:

The ImageAire Lite ii-2 UAV payload instrumentation is contained in a pod suspended from the center of the ImageAire Lite ii-2 wing.  This design isolates the Payload Electronics and Power Systems from the UAV Control and Propulsion systems.  The Payload Systems consist of the following:

1	Sony Color Microvideo Camera, 6mm lens, nadir view

1	Sony Monochrome Microvideo Camera, 6mm lens, 695 nm NIR filter, nadir view

1	FLIR Photon Thermal Infrared Camera, 19mm lens, nadir view

3	Micro Digital Video Recorders (DVR’s)

1	2.4 GHz Video Transmitter, FCC Part 15 Compliant

1	Miniature GPS Receiver w/Integrated Antenna

1	GPS Video Overlay Board

1	Madgetech PRHTemp101 Datalogger

1	Great Planes 3-cell/11.1V/910mAh Lithium-Polymer Battery

Imagery from the 3 video cameras is recorded onboard and downloaded after flight.  The color camera imagery is also downlinked via the 2.4 GHz video transmitter to provide the scientist with a realtime navigation reference.  This imagery may be recorded on the ground and may include GPS video overlay if desired.

The payload draws between 0.70 to 1.00 amps, depending on whether video is downlinked and GPS is turned on.  This gives a total payload runtime of  50 to 70 minutes, which must include payload preparation.  Battery time must be monitored to prevent discharging the LiPo battery below 9 volts or battery degradation will occur.

VIDEO SYSTEM GROUND STATION

The ImageAire Lite ii-2 video system ground station consists of a 2.4 GHz video receiver and a Sony Digital 8mm VCR to allow the scientist to observe the ground track during flight.  After charging batteries, connect the video output of the video receiver to the audio/video input jack on the Sony VCR using the supplied adapter and extension cable.  Turn both units ON, set the video receiver to channel 1 (2401 MHz) using the button on the side of the enclosure, insert tape in VCR, and observe video signal on the VCR LCD monitor.  When the Pilot is ready to launch the aircraft, the Payload Operator should start recording by pressing the RECORD button and the button immediately to the right simultaneously.  When recording is complete, press STOP to halt recording.

Video data may be analyzed post-flight by using a computer with a video capture card to “grab” visible images of ground features for comparison.

The maximum operating range of this system is approximately 2000 feet, which roughly corresponds to normal line-of-sight radio control flight.  An occasional video signal dropout is to be expected and should not affect data analysis.  If the plane is too far away to get a good video signal it is probably too far away to reliably control.

PILOT/OBSERVER COMMUNICATION

There is an observer standing within 10 ft of the pilot for the duration of the launch, flight and recovery.

- Pilot prepares for launch.

- Observer gives the signal that all is clear for launch.

- Pilot gives the command to count-down for launch.

- Observer counts down and hand launches aircraft into the wind.

- Observer directs pilot over the experimental area and warns pilot of any incoming aircraft or birds.

- Observer gives the command to end data collection and signals all is clear for landing.

- Pilot lands the aircraft as soon as it is safe to do so.