| Name | Description |
| --- | --- |
| RC Mode failsafe Loss Of Comm failsafe Loss of GPS Lock failsafe | If the autopilot does not receive RC controller packets for the specified amount of time, the autopilot switches to Manual Mode (holding aircraft level, cruise throttle). If communications are not re-established, stage 2 kicks in and the aircraft flies to Home or Rally. |
|  | Flies the aircraft Home or to Rally if communications are lost in any mode. |
|  | Puts the aircraft in a shallow bank if GPS lock is lost in Nav Mode, Home Mode, Rally Mode, or Loiter Mode. All of these modes rely on GPS lock to navigate the aircraft. As such, if GPS lock is lost the aircraft will “wander” and potentially become lost. By putting the aircraft in a shallow bank the distance the aircraft can wander is limited and the GPS is given the opportunity to re-acquire lock. If GPS lock is not re-acquired the user can program the autopilot to auto-land. |
| Low Battery failsafe Critically Low Battery failsafe Minimum Height Above Ground Flight Termination | Flies the aircraft Home or to Rally when the aircraft battery reaches a certain threshold of time as configured by the user. |
|  | Shuts the propulsion motor off and lands the aircraft at the current location. This failsafe is triggered if the battery drops below the critical battery voltage as configured by the user. Prevents the loss of an aircraft due to autopilot re-boot caused by low voltage |
|  | Changes desired altitude to that specified in the behavior column if the airplane flies below the specified trigger height above ground (HAG). This keeps the airplane from flying into the ground. This failsafe requires the use of terrain elevation |
|  | If communications is lost for the trigger time, all control surfaces go to pre-defined positions (i.e. to induce a flat spin) and to ground the aircraft. Used only during tuning before the autopilot is trusted to hold altitude and fly to waypoints. May also be used where flight termination devices are required. |

### 10.6.5 Broken Link

When communication is lost, the CyberBUG will enter a failsafe mode and navigate home. Meanwhile, initiate troubleshooting and try to reestablish the link. Check the GCS power, examine the status LEDs on the commbox, check computer and antenna connections.

If communication was lost beyond 3km, the CyberBUG probably reached the limits of its range. In failsafe mode, the CyberBUG will navigate towards home and communication will probably be restored when it comes into range. If communication is lost at close range, it may indicate a technical problem. Land ASAP if control is recovered.