Airworthiness assurance for RMAX flights at the Savannah River National Laboratory, Aiken, SC

- Essential Personnel

  - Mission Coordinator (MC)

    - The MC is responsible for ensuring all necessary personnel and equipment is available and present for flight operations.  The MC will make assignments, as necessary, for personnel to operate radios, video cameras and safety equipment such as fire extinguishers.

    - The MC will check weather conditions and confer with the pilot on acceptable / unacceptable conditions for flight.

    - The MC will monitor flight control frequencies during flight ops and will stand within 10 m of the pilot to communicate information.

    - The MC is responsible for the safe transit of the helicopter to the flight area.

    - The MC ensures that all necessary equipment for flight is transported to the flight area.  The MC can delegate responsibility to others.  A partial list includes:

      - Computers

      - Power (generator, power cords)

      - Antennas

      - Cameras

  - Pilot

    - The Pilot has primary responsibility for the operation, care and maintenance of the helicopter.

    - The Pilot is one of two responsible persons who will preflight the helicopter and complete a pre-flight check-list.  The Pilot’s pre-flight is conducted at the flying field just prior to engine start.

    - The Pilot will ensure that batteries are charged and ready for flight at the time agreed upon.

    - The Pilot will manage fuel for the helicopter.

    - The Pilot will maintain and update checklists as required – this particularly applies in the case of accidents leading to new items that must be checked to ensure airworthiness.

  - Ground control station (GCS) operator(s)

    - There will be a primary GCS person identified who will be responsible for equipment and operations set-up.

    - The GCS operator will set up the following equipment for flight:

      - Tables and awnings

      - Antennas

      - Computers

      - Generator and power distribution

- Handling and transporting helicopter

  - Whenever the helicopter or rotor blades are moved, two people must be present – no exceptions.

  - Carriage of the helicopter should be a two-person carry on each side of the landing gear with the blades stowed.

  - Before any action is taken to move the helicopter, the parties that are involved must confer with each other on the signals for pick-up, etc.

- Flight procedures

  - All necessary personnel will show up promptly to the lab at a time that allows 1 hour of preparation before departing the lab to fly.

  - Before leaving the lab with the helicopter, the following are accomplished:

    - One pre-flight will occur in the lab by a qualified inspector (not the pilot).  Using the aircraft checklist, all items are checked off as either satisfactory or unsatisfactory.

    - A briefing led by the MC will occur on how equipment will be set up and what the objectives of the mission are.

    - The MC will collect the 1st pre-flight form and confirm from the pilot that the helicopter has full fuel and batteries are charged

  - The MC will oversee the transportation of the helicopter and the set-up of the GCS.  The MC will ensure that the helicopter is carried and configured by two people at all times.  At no time should one person move the helicopter and/or deploy the blades.  This is always a two-person operation.

  - The following minimum equipment is required for flight operations:

    - 3 personnel (MC, GCS and Pilot) minimum, 4 preferred

    - Ground control station equipment

    - First aid kit

    - Fire extinguisher

    - Video camera

    - All services and equipment to support the helicopter flight and data acquisition

  - The pilot performs the final pre-flight check before engine start.  Only after the pre-flight checklist is turned in will the MC authorize engine start.

- Development of pre-flight inspection checklists

  - Pre-flight checklists are living documents.  They will be revised and added to as reliability information becomes available.  For instance, whenever a new failure mode is discovered, the system will be analyzed for the root cause of failure.  If it is determined that the failure mode is traceable to an inspectable component, that item will be added to the checklist.

  - The Pilot will be responsible for maintaining a current pre-flight checklist.

- Safety

  - Safety is a major concern for all stages of the flight operation.  Risks of injury associated with the airframe principally relate to the engine and rotor.  Lacerations, burns, fuel exposure, and blunt force injury are the major concerns; each of which are discussed below.  Treatments for these injuries are described in Section xx.

  - There are two phases of a flight operation that have unique risks which must be addressed:  start-up and hover.  During start-up, personnel are located close to the helicopter and are working with fuels, batteries and rotating equipment, all of which have unique hazards associated with them.  During hover, the helicopter is operating at full RPM so flight loads will be at their maximum.  A rotor blade that departs the aircraft can be thrown hundreds of feet, and so precautions to all ground personnel are necessary.

    - Start-up safety procedures

      - Ensure the helicopter is in a stable attitude (solid base) for start-up.  Dynamic transients during start-up can cause a rocking motion that may potentially lead to a rotor strike.

      - Personnel remain clear of moving parts, especially the rotor

      - A fire extinguisher will be located and manned within 50’ of the helicopter

      - During normal operation, engine components become quite hot and can easily burn the user on contact.  Precautions will be made to avoid contact with hot spots

      - While the rotor is spinning up, the nearest person to the helicopter will be the pilot.  The pilot should be at least 50’ away from the helicopter during this phase of the operation.  All other personnel should be at least 100’ from the helicopter.

    - Hover and in-flight safety procedures

      - After the helicopter takes off, the major danger during flight operations is loss of control of the aircraft.  All personnel will be vigilant of the helicopter flight path, and personnel will communicate flight status on a regular basis to inform on the health of the aircraft.

      - In the event of loss of control, the Pilot or GCS operator will inform everyone of the condition, at which time personnel will seek safe shelter.  Updated assessments by the Pilot and GCS operator will keep all personnel informed until helicopter control is regained, or the vehicle crashes.

  - Medical procedures

    - Lacerations.  A laceration is any wound where the skin is broken. The following procedures are for all sizes of lacerations.  If the wound is more then a half an inch long, is very deep, or is bleeding profusely, stitches might be required.  The first step to treating a laceration is to stop the bleeding. Apply constant pressure to the wound using a clean cloth or bandage. If the bleeding does not stop and it is possible to do so, raise the body part above the level of the victim's heart. If bleeding continues seek medical assistance.  To clean the wound, rinse it under cold water, however do not use soap directly on the wound. If there are debris in the wound, clean them out with a set of sterile tweezers.  Apply an antibiotic such as Neosporin and then dress the wound with a bandage. The bandage will keep the wound clean and prevent infection. Replace the bandage daily until the wound has healed enough to prevent infection. Once the wound has healed enough, remove the bandage and expose the wound to air which will speed the healing process.

    - Burns.  There are two kinds of burns that operators of UAVs should be aware of.  Heat burns are caused by contact with a hot surface, such as the engine of a UAV.  The second type of burn which may occur when operating a UAV is a chemical burn.  This may occur if a victim comes in contact with a toxic chemical, such as the fuel used in most UAVs.  If the toxic chemical comes in contact with the victim's eye, the eye should be thoroughly rinsed with water and medical attention should be sought.  There are 3 degrees of heat burns.  First degree burns are the least severe, leaving a red spot on the victim's skin.  Second degree burns resemble first degree burns, but the victim will also have blisters on their skin.  Third degree burns are the most severe and the victim will have a deep wound with charred skin.  Although there is little or no pain with a third degree burn, such wounds are quite severe and require careful attention.  The following procedures apply to chemical burns, first degree burns, and small (under 2 inch diameter) second degree burns. For a second degree heat burn that is larger then two inches or for a third degree heat burn, seek medical attention.  If any of the victim's clothing is still smoldering or came in contact with the chemical, remove it.  Cooling the burn will stop the burning process and remove chemicals left from contact. To cool the burn, run the affected area under cool water for fifteen minutes.  Bandaging the burnt area will keep the air off the burn and lessen the pain. When bandaging the burn, wrap a sterile gauze strip around the affected area. Do not use any cotton bandages, because these might stick and irritate the skin.

    - Vapor exposure.  When operating a UAV it is possible to inhale harmful vapors from the engine or engine fuel.  Symptoms of light vapor inhalation are drowsiness and inability to think clearly.  More serious exposure may lead to vomiting or unconsciousness.  For serious vapor exposures, seek medical assistance immediately.  Information about the fuel being used should be on hand to aid emergency crews.  The following procedures are appropriate for treating the symptoms of light vapor inhalation.  Remove the victim from the vapor source as quickly as possible.  Make sure the area is well ventilated so the victim can get fresh air.  Keep the victim calm and have him or her lie down to reduce their breathing rate.  Loosen tight clothing around the neck to aid breathing.  If victim's condition does not improve, seek medical assistance.

    - Blunt Force Injuries.  When operating a helicopter, there is a remote chance of being struck by a rotor blade.  In blunt force injuries, the blood vessels under the skin are broken, resulting in a blue or black colored skin. If the skin is broken, follow the procedures for lacerations described in 5.3.1.  Elevating the injured area will reduce blood flow to the wound and reduce pain.  Apply ice or a cold pack to the injured area for twenty to thirty minutes at a time for a few days afterwards. This will reduce swelling and pain.  Seek medical assistance if the pain is excessive or lasts for more then a few days.

Mission coordinator checklist

Mission Coordinator: ______________________________

Date: _______________________

Item	Accomplished?

- Weather check	Winds: ____________________________

Temperature: _______________________

Barometric Pressure: _________________

- All essential personnel present for briefing?	Yes / No

- Briefing completed?	Yes / No

- First pre-flight check has been completed?	Yes / No (collect)

- Pilot has charged batteries?	Yes / No

- Pilot has fueled helicopter?	Yes / No

- All equipment loaded, transported and set up?	Yes / No

- GCS is set up and operational?	Yes / No

- Pilot has completed the 2nd pre-flight inspection?	Yes / No (collect)

- Frequency monitor shows no interfering signals?	Yes / No

- Cameras are set up for video?	Yes / No

- Start time of mission: ____________________________

- End time of mission: ____________________________

Notes on flight:

| RMAX Flight Checklist |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  | Starting Flight Time |  |  |
| Time |  |  |  | Finishing Flight Time |  |  |
| Weather |  |  |  | Length of Flight |  |  |
| Temperature |  |  |  | Total Flight Hours on Helicopter |  |  |
| Altitude |  |  |  | Location |  |  |
|  |  |  |  | Pilot |  |  |
| Preflight Check |  |  |  | Flight Controls Check |  |  |
| Item | Check | Initial |  | Item | Check | Initial |
| Engine Seals | Leaks |  |  | Cyclic Roll | Direction |  |
| Carburetor | Clean |  |  | Cyclic Pitch | Direction |  |
| Air Filter | Clean |  |  | Collective | Direction |  |
|  | Condition |  |  | Throttle | Play |  |
| Main Rotor Head | Ball Links |  |  |  | Direction |  |
|  | Play |  |  |  | Return |  |
|  | Condition |  |  | Rudder | Direction |  |
| Main Rotor Blade Fastener | Proper torque |  |  | Gyro | Direction |  |
| Main Rotor Blades | Condition |  |  | Servos | Noise |  |
| Stabilizer | Play |  |  |  | Smooth Operation |  |
|  | Condition |  |  | Range Check(50m) | Distance |  |
| Stabilizer Blades | Condition |  |  |  |  |  |
| Tail Rotor Head | Play |  |  | On First Flight |  |  |
|  | Condition |  |  | Item | Initial |  |
| Tail Rotor Blade Fastener | Proper torque |  |  | Tracking |  |  |
| Tail Rotor Blades | Condition |  |  | Trim |  |  |
| Tail Rotor Drive Belt | Condition |  |  | Performance/Sound |  |  |
|  | Tension |  |  |  |  |  |
|  | Lubrication |  |  | Post Flight |  |  |
| Main Gear | Condition |  |  | Item | Initial |  |
|  | Play |  |  | Receive Off |  |  |
| Fuel Tank | Mixture |  |  | Tramsmitter Off |  |  |
|  | Level |  |  | Wipe Down Inspection |  |  |
| Servo Batteries | Voltage Level |  |  |  |  |  |
|  | Battery Condition |  |  |  |  |  |